TRIBHUVAN UNIVERSITY INSTITUTE OF ENGINEERING PASHCHIMANCHAL CAMPUS

PROJECT REPORT ON
DESIGN AND DEVELOPMENT OF A LOW-COST VIBRATION MONITORING SYSTEM FOR ELECTRICAL MOTOR ANALYSIS

SUBMITTED BY:
ARYAN PARAJULI (PAS080BME003)
AAYUSH BHANDARI (PAS080BME003)
KIRAN BHUSAL (PAS0080BME023)
SUSHIL NYAUPANE (PAS080BME042)

SUPERVISED BY
Ghanshyam Aryal

Submitted to:
DEPARTMENT OF MECHANICAL AND AUTOMOBILE ENGINEERING
IOE, PASCHIMANCHAL CAMPUS
LAMACHAUR-16, POKHARA

AUGUST 2026

# ABSTRACT

This project presents an experimental study of motor vibration characteristics under different controlled operating and mechanical conditions using an ADXL345 three-axis accelerometer interfaced with an ESP32-based data acquisition system. Vibration acceleration data are acquired from a DC motor–generator setup along the X_g, Y_g, and Z_g axes and processed to obtain suitable signals for vibration analysis.

The experimental investigation considers normal operating conditions, controlled shaft unbalance, variable generator loading, controlled mechanical looseness, and different motor speeds. For each condition, vibration data are collected systematically while maintaining consistent measurement and experimental parameters. The acquired signals are analyzed using both time-domain and frequency-domain techniques to characterize changes in motor vibration behavior.

Relevant vibration features, including RMS, peak amplitude, peak-to-peak value, standard deviation, crest factor, kurtosis, skewness, dominant frequency, and spectral energy, are extracted from the measured signals. Further analysis is performed using Fast Fourier Transform (FFT), Power Spectral Density (PSD), spectrogram, autocorrelation, cumulative distribution, and spectral analysis.

The study aims to establish an experimental relationship between controlled motor operating conditions and measurable changes in vibration characteristics. The resulting datasets and extracted features provide a quantitative basis for comparing motor vibration behavior across different experimental conditions and contribute to the study of vibration-based condition monitoring techniques.

Keywords: Accelerometer, Motor Vibration, Signal Analysis, Vibration Features, Vibration Monitoring.

# Acknowledgement

We would like to express our sincere gratitude to our project supervisor, Er. Ghanshyam Aryal, for his valuable guidance, continuous support, constructive feedback, and encouragement throughout the completion of this project. We would also like to extend our special thanks to Asst. Prof. Suraj Shrestha, Head of the Department of Electrical Engineering, IOE, Paschimanchal Campus, for his valuable support, encouragement, and for providing the necessary academic environment and resources for this project.

We are equally grateful to all the faculty members, staff, and colleagues of IOE, Paschimanchal Campus who directly or indirectly contributed to the successful completion of this work. Finally, we would like to thank our families and friends for their constant encouragement, patience, and support throughout the project.

Thank you all for your invaluable support and contribution to the successful completion of this project.

Table of Content

ABSTRACT	ii

Acknowledgement	iii

List of Figures	vi

List of Tables	vii

Chapter One: Introduction	1

1.1. Background	1

1.2. Problem Statement	2

1.3. Scope of Work	3

1.4. Objectives:	5

Main Objective	5

1.5. Limitations	6

Chapter Two: Literature Review	7

Chapter Three: Materials and Methodology	9

3.1 Materials:	9

Hardware Components	9

Software Components	15

3.2 Methodology	16

3.2.1 Overall Methodology	16

3.2.2 Accelerometer Mounting and Orientation	19

3.2.3 Vibration Data Acquisition	19

3.2.4 Sensor Configuration and Sampling	20

3.2.5 Data Storage and Dataset Preparation	21

3.2.6 Signal Preprocessing	21

3.2.7 Time-Domain Analysis	22

Chapter Four: Time frame of work	23

Chapter Five: Expenses	24

Chapter Six: RESULTS AND ANALYSIS	25

6.1 Normal Condition Vibration Analysis.	25

6.1.1 Spectral characteristics of the normal condition	26

6.1.2 Dominant frequency behaviour in individual axes	26

6.1.3 Resultant vibration behavior	27

6.1.4 Main findings from normal-condition testing	28

6.2 Analysis of Loaded-Normal Condition	34

6.2.1 Running-speed component	34

6.2.2 Comparison with the normal unloaded condition	34

6.3 Mechanical Looseness	40

6.3.1 Normal baseline vs mechanical looseness: dominant amplitude	40

6.3.2 Change relative to normal baseline	40

6.3.2 Dominant frequencies for mechanical looseness	40

6.3.3 Graphs and Plots of Mechanical Looseness:	41

6.3.4 Findings and Discussion – Mechanical Looseness	42

6.4 Sudden Load	45

6.4.1 Time-domain features	45

6.4.2 Statistical Features	45

6.4.3 Frequency Domain Features	45

6.4.4 Spectral Features	46

6.4.5 Findings	46

6.4.6 Conclusion	48

6.4.7 Plots and Graph	49

6.5 Load Imbalance	53

6.5.1 Baseline for Load Imbalance	53

6.5.2 Normal-condition baseline characterization	56

6.5.3 Load Imbalance Condition Analysis	57

6.5.3.1 Time-Domain Comparison	57

6.5.3.2 RMS vibration level for baseline and load-imbalance samples	58

6.5.3.3 Change in RMS vibration relative to the normal baseline	58

Chapter Seven: Conclusion	63

References	64

APPENDIX I	65

Appendix II	107

# List of Figures

Figure 1 ADXL345	9

Figure 2 ESP 32 MCU	10

Figure 3 Electric Load Rig	12

Figure 4 Bench Power Supply	13

Figure 5 Project FlowChart	15

Figure 6 Sensor Mounting	16

Figure 7 Gantt Chart	19

Figure 8 Normal FFT	25

Figure 9 Normal Autocorrelation	25

Figure 10 Normal CDF	25

Figure 11 Local Spectral Analysis 1	26

Figure 12  Local Spectral Analysis 2	26

Figure 13  Local Spectral Analysis 3	26

Figure 14 PSD 1	27

Figure 15 PSD 2	27

Figure 16 PSD 3	28

Figure 17 PSD 1 Zoom	28

Figure 18 PSD 2 Zoom	28

Figure 19 Graphs and Plots of Loaded Condition2:	32

# List of Tables

Table 1 ADXL 345 Specs	9

Table 2 MCU Specification	10

Table 3 DC Generator Specification	11

Table 4 Software Components	14

Table 5 Sensor Interfacing	17

Table 6 Expenses	20

Table 7 Normal Features1	21

Table 8 Normal Features 2	22

Table 9  Normal Features 3	23

Table 10  Normal Features 4	23

Table 11: Normal Findings And Observations	25

Table 12 Running-speed component of Normally Loaded	29

Table 13 Comparison	30

# Chapter One: Introduction

## 1.1. Background

Electric motors are widely used in industrial and mechanical systems for applications such as pumping, manufacturing, transportation, material handling, and power transmission. During operation, mechanical forces, rotational motion, electromagnetic effects, and variations in operating conditions produce vibrations in the motor and its associated components. The characteristics of these vibrations can change with changes in motor operating and mechanical conditions.

Vibration measurement provides a non-invasive method for studying the dynamic behavior of rotating machinery. An accelerometer can measure the acceleration produced by mechanical vibrations, allowing the measured signal to be analyzed quantitatively. In a three-axis measurement system, vibration can be observed along different directions, providing a more complete representation of the motor's dynamic response.

In this project, an ADXL345 three-axis accelerometer interfaced with an ESP32 is used to acquire vibration data from a DC motor–generator setup. The measured acceleration is obtained along the X_g, Y_g, and Z_g axes. The acquired data are calibrated and processed before further analysis to reduce measurement offsets and obtain suitable vibration signals.

A single numerical quantity does not represent motor vibration. Its characteristics can be examined in both the time domain and frequency domain. Time-domain parameters such as RMS, peak amplitude, peak-to-peak value, crest factor, kurtosis, and skewness provide quantitative descriptions of the vibration signal's amplitude and statistical behavior. Frequency-domain methods such as Fast Fourier Transform (FFT), Power Spectral Density (PSD), spectral energy, and spectral analysis provide information about how vibration components are distributed with frequency.

The vibration behavior of a motor can varies with its operating and mechanical conditions. Therefore, controlled experiments involving normal operation, shaft unbalance, generator loading, mechanical looseness, and different operating speeds provide an opportunity to study these changes systematically. By collecting repeated datasets under controlled conditions and extracting consistent vibration features from each dataset, the variation in motor vibration characteristics can be quantitatively investigated.

This experimental approach provides the foundation for studying the relationship between motor operating conditions and measured vibration characteristics using sensor-based data acquisition and digital signal processing.

## 1.2. Problem Statement

Electric motors are critical components in industrial, power-generation, and domestic applications, where their reliable operation is essential for maintaining continuous mechanical and electrical processes. As electromechanical systems, motors are subjected to friction, wear, mechanical loading, and other stresses during operation. These effects can alter the dynamic behavior of a motor and may be reflected in measurable changes in its vibration characteristics.

Vibration-based condition monitoring provides a practical approach for observing these changes because motor operating and mechanical conditions can produce distinguishable variations in measured vibration signals. However, conventional industrial vibration monitoring systems often rely on high-precision sensors, dedicated data-acquisition hardware, and specialized software, making such systems comparatively expensive and complex for small-scale industries, educational institutions, and research laboratories.

The need for an accessible experimental platform therefore exists for acquiring and quantitatively analyzing motor vibration data under controlled conditions. In particular, there is a need to understand how measurable vibration characteristics vary when motor operating and mechanical parameters are changed in a controlled experimental environment.

This project addresses this requirement by developing a low-cost motor vibration measurement and analysis system using an ADXL345 three-axis accelerometer interfaced with an ESP32 and Python-based signal processing. A DC motor coupled to a DC generator is used as the experimental platform. Vibration data are acquired along the X_g, Y_g, and Z_g axes under controlled conditions including normal operation, controlled rotor unbalance, variable generator loading, controlled mechanical looseness, and different motor speeds.

The acquired data are processed and analyzed using time-domain and frequency-domain techniques to extract measurable vibration characteristics such as RMS, peak amplitude, peak-to-peak amplitude, standard deviation, crest factor, kurtosis, skewness, dominant frequency, and spectral energy. FFT, Power Spectral Density, spectrogram, autocorrelation, and other signal-analysis techniques are also employed to examine the vibration signals.

The resulting experimental dataset provides a quantitative basis for studying how motor vibration characteristics change with controlled operating and mechanical conditions, while establishing a low-cost experimental platform that can serve as a foundation for further motor condition-monitoring research and future automated analysis applications.

## 1.3. Scope of Work

The scope of this project encompasses the development and experimental evaluation of a low-cost motor vibration measurement and analysis system using an ADXL345 three-axis accelerometer, ESP32 microcontroller, and Python-based signal processing tools. The study focuses on experimentally investigating how controlled mechanical and operating conditions influence the vibration behaviour of a DC motor relative to its normal baseline condition.

The project includes the development of a structured vibration data acquisition and processing system. The ADXL345 accelerometer is interfaced with the ESP32 to acquire acceleration along three orthogonal axes. Sensor calibration and offset correction are performed to improve measurement consistency and reduce the effect of sensor bias. The acquired measurements are transferred to a computer and processed to obtain calibrated acceleration signals represented by X_g, Y_g, and Z_g. The developed acquisition system operates at an experimentally measured sampling frequency of approximately 1559 Hz.

A DC motor mechanically coupled to a DC generator is used as an experimental platform. A normal operating condition is established as the baseline reference, followed by controlled variations in the motor's mechanical and operating conditions. The experimental investigation covers controlled rotor unbalance, variable generator loading, controlled mechanical looseness, and different motor speeds. For the unbalanced condition, a known eccentric mass is securely positioned at a measured radial distance from the motor shaft. Generator loading varies through its electrical load to modify the mechanical loading of the motor, while controlled mounting conditions are used to investigate the effect of mechanical looseness. Different operating speeds are also considered to examine their influence on vibration behaviour.

Multiple vibration measurements are collected for each experimental condition to establish a structured dataset. The acquired signals are processed consistently across all conditions to allow meaningful comparison with the baseline measurement. The study focuses on identifying observable changes in vibration behaviour resulting from the introduction of each controlled condition, rather than relying only on individual numerical features.

The processed X_g, Y_g, and Z_g signals are analysed in both the time and frequency domains. Time-domain characteristics including RMS, peak amplitude, peak-to-peak amplitude, standard deviation, variance, crest factor, kurtosis, skewness, and other relevant statistical features are extracted to quantify changes in signal amplitude and distribution. Frequency-domain analysis includes Fast Fourier Transform (FFT), dominant frequency analysis, spectral energy, and Power Spectral Density (PSD) to examine changes in frequency content and energy distribution.

Additional signal-analysis techniques, including spectrograms, autocorrelation, cumulative distribution functions, and spectral analysis, are used to examine changes in waveform structure, periodicity, frequency components, and vibration-energy distribution. Time-domain and frequency-domain plots are generated for systematic visual comparison between the baseline and controlled experimental conditions.

The extracted features and signal characteristics are compared across the experimental conditions to identify characteristic changes associated with controlled unbalance, generator loading, mechanical looseness, and speed variation. Particular attention is given to changes in vibration amplitude, dominant frequency components, harmonic content, spectral-energy distribution, waveform characteristics, and statistical features. The experimental observations are based on the measured data and are used to describe how different controlled conditions manifest themselves in the motor's vibration response.

The project establishes a low-cost experimental framework for motor vibration measurement, signal processing, feature extraction, and condition-based comparative analysis. The resulting experimental dataset and observations provide a foundation for understanding motor vibration behavior and for further development of vibration-based condition monitoring, fault identification, and predictive maintenance techniques.

## 1.4. Objectives:

## Main Objective

To experimentally investigate the vibration behavior of a DC motor under controlled mechanical and operating conditions using an ADXL345 three-axis accelerometer, and to characterize changes in vibration behavior relative to a normal baseline through time-domain and frequency-domain analysis.

Specific Objectives

- To develop and implement a low-cost vibration data acquisition system using an ADXL345 accelerometer and ESP32 for recording three-axis motor vibration signals (X_g, Y_g, and Z_g).

- To develop and implement a suitable sensor mounting arrangement for securely attaching the accelerometer to the motor housing and obtaining consistent vibration measurements.

- To calibrate and process the acquired accelerometer data to reduce sensor offset and obtain reliable vibration signals for subsequent analysis.

- To experimentally collect and organise vibration data from the DC motor under a normal baseline condition and controlled mechanical and operating conditions, including rotor unbalance, variable generator loading, mechanical looseness, and different motor speeds.

- To perform time-domain and frequency-domain analysis of the acquired vibration signals using techniques including waveform analysis, FFT, Power Spectral Density (PSD), spectrogram, autocorrelation, cumulative distribution, and spectral-energy analysis.

- To extract relevant vibration characteristics and features such as RMS, peak amplitude, peak-to-peak amplitude, standard deviation, variance, crest factor, kurtosis, skewness, dominant frequency, and spectral energy from the measured signals.

- To compare the vibration response of each controlled condition with the normal baseline and investigate changes in vibration amplitude, waveform characteristics, frequency components, harmonic content, spectral-energy distribution, and statistical features.

- To identify and document characteristic vibration patterns associated with the experimentally induced conditions, based on measured data, and establish an experimental basis for understanding how different mechanical and operating conditions manifest in motor vibration signals.

## 1.5. Limitations

- The developed data acquisition system has an experimentally measured sampling frequency of approximately 1559 Hz, limiting reliable frequency-domain analysis to below approximately 779.5 Hz.

- The ADXL345 provides lower measurement resolution, sensitivity, and noise performance than dedicated industrial vibration sensors.

- The accuracy and consistency of acquired vibration data depend on the sensor mounting position, orientation, and mechanical attachment.

- The system records three-axis acceleration (X_g, Y_g, and Z_g) and does not directly acquire parameters such as rotational speed, phase, torque, temperature, or electrical current.

- The processing system relies on the quality and consistency of the acquired signals. Sensor noise, electrical interference, missing samples, and other acquisition artefacts can influence extracted features and spectral characteristics.

- The current processing framework analyses the acquired vibration data offline and therefore does not provide real-time automated processing and visualisation during data acquisition.

- The reliability of frequency-domain features depends on the selected sampling frequency, measurement duration, windowing method, and signal-processing parameters.

- The system's vibration analysis is restricted to the frequency range adequately captured by the ADXL345 and the selected acquisition and sampling configuration.

# Chapter Two: Literature Review

In the real-world scenario, nothing works in an ideal case. Every system is affected by real-world parameters. Similarly, in functional motors, load imbalance, shaft misalignment, bearing fault and mechanical looseness cause inefficiency in motors. A lot of research has been done into the diagnosis of these faults.

Similarly, (Quan et al., 2025) in their work introduced an embedded motor fault diagnosis system utilising vibration signal analysis and machine learning. The system is built using an STM32H753 microcontroller, which gathers real-time vibration signals from the motor using a vibration sensor. Those signal features are extracted using Discrete Wavelet Transform (DVT) and combined with a machine learning anomaly detection (AD) model for real-time automatic monitoring of motor health status and fault identification. FIR (Finite Impulse Response) filters are used as digital filters. They used data mining techniques to identify abnormal data points that don’t fit the overall pattern of the dataset.

Likewise, bearing fault is also a major issue seen in motors. It is crucial and a significant concern. In this article by Hsia, they presented a design of a vibration measurement system accompanied by a control system using a Field Programmable Gate Array (FPGA). The vibration signals are captured and processed in the same way using fast Fourier transform (FFT) (Hsia & Hsu, 2024) with hardware Implementation, enabling real-time display of both the time domain and frequency domain signals. They performed the motor speed control system through logical analysis. A self-adjusting speed method is integrated to mitigate abnormal vibration anomalies. Their measurement program control system facilitates quick data collection of vibration and speed patterns of the motor for identifying abnormal data patterns. The measurement results deliberately illustrate the real-time detection of motor fault types and motion vibrations.

In this research article, they have used the common Data Acquisition (DAQ) technique, measuring time- and frequency-domain responses. Collecting samples of datasets from healthy motors on a grid. During the analysis of fault motors, the vibration pattern changes. Basically, it only covers the analysis of the conditions of motors in healthy, unbalanced, mechanical looseness, misalignment, bent shaft, broken bar, and bearing fault conditions.  (Ribeiro Junior et al., 2021) The proposed methodology is verified through a real experimental setup

Cost efficiency is a major factor in all research purposes. Hence, to maintain a cost-effective background the low-cost real-time monitoring of electric motors for the industry of 4.0, Magadan has made a cost-effective real-time monitoring system. (Magadán et al., 2020) It uses wireless multi-sensor modules and a single-board computer as a gateway, open-source software and a free version of an IoT analytics service in the cloud, where all the relevant information is stored. This module is unique in a way that it not only takes vibrational data, but also takes temperature and thermal data from the monitored motor. The frequency domain analysis is carried out both in the module and gateway to compare their capabilities. This approach can be considered as a springboard to take advantage of edge and fog computing as a complement to cloud computing. They have successfully tested the prototype in laboratory conditions and an industrial dairy plant.

In the manufacturing process, the principle of Predictive Maintenance (PdM) is widely used to reduce maintenance costs and unexpected downtime. A motor, being a common element within manufacturing equipment/machines, this system is used to detect and analyse motor faults using an Artificial Intelligence (AI) model trained using the collected vibration data, and principal component analysis (PCA) is used to detect abnormal behaviours of the motor. (Joung et al., 2020) They used Hostelling’s T2 statistics and squared prediction error (SPE) statistics to clarify criteria for abnormal operations of the motor.

Existing studies have demonstrated the effectiveness of vibration analysis, FFT, FPGA systems, IoT-based monitoring, and machine learning techniques for motor fault diagnosis. However, many of these approaches rely on specialised hardware, expensive data acquisition systems, or complex architectures. Therefore, there remains a need for a low-cost, easily deployable solution that combines vibration sensing, feature extraction, and machine learning-based diagnosis using readily available hardware such as Arduino and ADXL345 sensors.

# Chapter Three: Materials and Methodology

## 3.1 Materials:

The experimental system consisted of hardware and software components for three-axis vibration measurement, data acquisition, signal processing, feature extraction, and comparative analysis of a DC motor under different operating and mechanical conditions.

### Hardware Components

- A 2.4 kW DC motor was used as the test machine for the experimental investigation of DC motor vibration behaviour. The motor had a rated voltage of 220 V, rated current of 10 A, and rated rotational speed of 1420 rpm. The motor was operated under controlled experimental conditions, and the vibration response of its housing was measured using the ADXL345 accelerometer.

The motor served as the primary mechanical source for the vibration signals analysed in this study. Different operating and mechanical conditions were established during the experiments, and the resulting three-axis acceleration signals were acquired to investigate changes in the motor's vibration characteristics. The motor was also mechanically coupled to a DC generator, which enabled controlled variation of the mechanical load during the experiments. The rated rotational speed of 1420 rpm corresponds to approximately 23.67 Hz in rotational frequency. This rotational frequency provides a useful reference when examining the frequency-domain vibration response of the motor. However, the actual operating speed during each experiment was not assumed to be exactly equal to the rated speed; therefore, measured operating speed should be considered when associating observed spectral components with rotational frequency or its harmonics.

- ADXL345 Accelerometer

The ADXL345 is a low-power, three-axis MEMS (Micro-Electro-Mechanical Systems) accelerometer capable of measuring static and dynamic acceleration along the X, Y, and Z axes. It provides digital acceleration measurements and is designed for applications involving motion, tilt, shock, and vibration measurement. The sensor provides selectable measurement ranges of ±2 g, ±4 g, ±8 g, and ±16 g and supports both normal and full-resolution operating modes. In full-resolution mode, the sensitivity is maintained at approximately 3.9 mg/LSB, allowing acceleration measurements to be represented with consistent scale across the available measurement ranges. The ADXL345 supports digital communication through SPI and I²C interfaces, making it suitable for direct connection to microcontrollers. It also incorporates a 32-level FIFO buffer for temporary storage of acceleration samples and provides interrupt outputs for sensor events and data-related conditions. The device operates from a supply voltage in the low-voltage range and supports output data rates from low-frequency measurements up to a nominal maximum of 3200 Hz, depending on the selected configuration. Its combination of three-axis measurement, digital communication, selectable measurement range, and relatively high output data rate makes it suitable for vibration measurement applications.

Table 1 ADXL 345 Specs

| Parameter | Specification |
| --- | --- |
| Sensor type | 3-axis MEMS accelerometer |
| Measurement axes | X, Y, Z |
| Measurement range | ±2 g, ±4 g, ±8 g, ±16 g |
| Resolution | 13-bit maximum |
| Full-resolution sensitivity | Approximately 3.9 mg/LSB |
| Communication interface | SPI / I²C |
| FIFO | 32-level |
| Maximum output data rate | 3200 Hz |
| Interrupt outputs | INT1, INT2 |
| Supply voltage | 2.0–3.6 V |

Figure 1 ADXL345

- ESP32 Development Board

The ESP32 is a 32-bit microcontroller-based development platform developed around the ESP32 system-on-chip (SoC). It integrates a dual-core processor, digital input/output interfaces, communication peripherals, timers, and memory resources within a single device. The ESP32 supports high-speed digital communication interfaces including SPI, I²C, UART, and other peripheral interfaces, making it suitable for interfacing with digital sensors and data acquisition devices. The ESP32 operates at a logic level of 3.3 V and provides sufficient processing capability for high-rate sensor data handling. Its SPI peripheral supports high-speed communication with external devices, while its USB-to-serial interface enables communication between the microcontroller and a computer. The development board also provides multiple programmable GPIO pins, allowing external sensors, communication interfaces, and other peripherals to be connected. Its combination of processing capability, communication interfaces, GPIO availability, and 3.3 V operation makes it suitable for embedded data acquisition and sensor-based measurement applications.

| Parameter | Specification |
| --- | --- |
| Microcontroller | ESP32 |
| Processor architecture | 32-bit |
| Processor cores | Dual-core |
| Operating logic voltage | 3.3 V |
| Wireless connectivity | Wi-Fi, Bluetooth |
| SPI interface | Available |
| I²C interface | Available |
| UART interface | Available |
| GPIO | Multiple programmable GPIO pins |
| USB interface | USB-to-serial communication |
| Operating frequency | Up to 240 MHz |

Figure 2 ESP 32 MCU

Table 2 MCU Specification

- DC Generator

The DC generator used in the experimental setup had specifications comparable to the test motor, with a rated voltage of 220 V, rated current of 10 A, rated power of 2.4 kW, and rated speed of 1420 rpm. The generator is an electromechanical energy-conversion device that converts mechanical energy supplied through its shaft into electrical energy. In the experimental arrangement, it served as the loading machine for the DC motor. By varying the electrical load connected to the generator, the mechanical load imposed on the motor could be varied, allowing the motor's vibration response to be studied under different loading conditions.

| Parameter | Specification |
| --- | --- |
| Type | DC generator |
| Rated voltage | 220 V |
| Rated current | 10 A |
| Rated power | 2.4 kW |
| Rated speed | 1420 rpm |
| Function | Mechanical loading of DC motor |
| Output | DC electrical power |

Table 3 DC Generator Specification

- Electrical Load

The electrical load was connected to the DC generator to provide controlled variation of the mechanical load applied to the test motor. The electrical loading of the generator determines the electromagnetic torque opposing its rotation, thereby changing the torque demand on the motor. Different loading levels were established by varying the connected electrical load, allowing the vibration response of the motor to be investigated under different operating conditions. The loading arrangement provided a practical method for producing controlled changes in motor operating conditions while maintaining the same motor-generator mechanical coupling.

| Parameter | Specification |
| --- | --- |
| Type | Variable electrical load |
| Function | Application of mechanical load to motor through DC generator |
| Load adjustment | By varying connected electrical load |
| Input | Electrical output from DC generator |
| Effect | Variation of generator torque and motor loading |

Figure 3 Electric Load Rig

- Bench Power Supply

The DC power supply was used to provide the required electrical power for the experimental motor-drive arrangement. It provides a controlled DC electrical source to the motor, allowing the motor to operate at the required operating condition during vibration measurements. The supply was selected to provide adequate voltage and current capacity for the rated requirements of the motor and experimental setup.

Figure 4 Bench Power Supply

- Computer

A personal computer was used as the main processing and data-storage platform for the vibration measurement system. It received the acceleration data transmitted by the ESP32 through the serial communication interface and provided the computational environment required for data storage, preprocessing, signal processing, feature extraction, visualisation, and analysis.

### Software Components

The software components were used for data acquisition control, data storage, signal processing, feature extraction, visualisation, and analysis of the measured vibration signals. The software environment consisted primarily of Python-based tools, which provided the computational framework for processing the three-axis acceleration data obtained from the experimental setup.

| Software | Purpose |
| --- | --- |
| Python | Data processing, signal analysis and feature extraction |
| Pandas | Data handling and CSV dataset processing |
| NumPy | Numerical computation and array-based signal processing |
| Matplotlib | Graphical visualisation of vibration signals and analysis results |
| SciPy | Signal processing, FFT, PSD and related numerical analysis |
| Report Lab | Automated generation of analysis reports in PDF format |
| Arduino/ESP32 development environment | Programming and configuration of the ESP32 data acquisition system |

Table 4 Software Components

## 3.2 Methodology

### 3.2.1 Overall Methodology

The experimental methodology was developed to investigate the vibration behaviour of a DC motor under controlled operating and mechanical conditions using a three-axis accelerometer-based data acquisition system. A 2.4 kW, 220 V DC motor rated at 1420 rpm was used as the test machine, and an ADXL345 accelerometer was mounted on the motor housing to measure vibration along the X, Y, and Z axes.

The experimental investigation consisted of establishing a baseline vibration condition followed by controlled introduction of selected changes in the motor operating or mechanical condition. The vibration signals corresponding to each condition were acquired using the developed ESP32-based data acquisition system. The acquired measurements were stored as digital three-axis acceleration data and subsequently processed using Python-based signal-processing routines.

The processing procedure included data validation, calibration, preprocessing, time-domain analysis, and frequency-domain analysis. Statistical and vibration-related features were extracted from the processed signals, while FFT- and PSD-based analyses were used to investigate the frequency characteristics of the measured vibration. The characteristics obtained under the different experimental conditions were then compared with the corresponding baseline behavior to identify changes in vibration magnitude, statistical characteristics, and spectral content.

The methodology therefore provides a systematic experimental framework in which changes introduced to the motor condition can be related to measurable changes in its vibration response.

Figure 5 Project Flowchart

### 3.2.2 Accelerometer Mounting and Orientation

The ADXL345 accelerometer was mounted directly onto the motor housing using a strong adhesive to establish a firm mechanical connection between the sensor and the motor structure. The sensor was positioned as close as practically possible to the motor shaft in order to measure the vibration transmitted through the motor housing with minimal influence from distant structural locations. The accelerometer was maintained in a fixed orientation throughout the experimental measurements, with its three sensing axes corresponding to the X, Y, and Z directions. Maintaining a consistent sensor position and orientation was important for obtaining comparable vibration measurements between the baseline and the different experimental conditions. The mounting location was selected on the motor housing near the shaft-bearing region because vibration generated by rotating and mechanical components is transmitted through the motor structure. Positioning the sensor near this region provides a direct measurement of the local structural vibration response. For the experiments, the sensor was not repeatedly repositioned between operating conditions. The same mounting location and orientation were maintained to minimise measurement variation caused by changes in sensor placement.

Figure 6 Sensor Mounting

### 3.2.3 Vibration Data Acquisition

The vibration data acquisition system was developed using an ADXL345 three-axis accelerometer and an ESP32 microcontroller. The accelerometer measured the vibration response of the motor housing along the X, Y, and Z axes and generated digital acceleration samples for each axis. The ADXL345 was configured for a nominal output data rate of 1600 Hz and operated in the ±2 g full-resolution mode. Communication between the accelerometer and ESP32 was established through the SPI interface. The sensor's internal FIFO was configured in stream mode to buffer acquired samples before they were read by the ESP32.

ESP32 retrieved the buffered X, Y, and Z acceleration samples and transmitted the measurements to a computer through a high-speed serial connection. The serial data were organised as three-axis measurements in the form. Each recorded sample therefore represented the instantaneous acceleration measured along the three sensor axes. The acquired data were subsequently stored in CSV format for calibration, preprocessing, feature extraction, and vibration analysis. The acquisition system provided an experimentally measured sampling rate of approximately 1559 Hz, compared with the ADXL345's nominal configured output data rate of 1600 Hz. This measured sampling rate was used in subsequent frequency-domain calculations, including determination of the Nyquist frequency and frequency resolution. The Acquisition ESP32 code and Python code are given in the appendix section.

| ADXL345 Pin | ESP32 Pin | Function |
| --- | --- | --- |
| VCC | 3.3V | Power supply |
| GND | GND | Ground |
| CS | GPIO 5 | SPI Chip Select |
| SDO | GPIO 19 | SPI MISO |
| SDA | GPIO 23 | SPI MOSI |
| SCL | GPIO 18 | SPI Clock |
| INT1 | Not connected | Interrupt output |
| INT2 | Not connected | Interrupt output |

Table 5 Sensor Interfacing

### 3.2.4 Sensor Configuration and Sampling

The ADXL345 accelerometer was configured in full-resolution mode with a measurement range of ±2 g. The nominal output data rate was set to 1600 Hz to provide a sufficiently high sampling rate for capturing the vibration response of the motor. The accelerometer was operated using the SPI communication interface with the ESP32. The sensor's internal FIFO was configured in stream mode to buffer successive three-axis measurements. This allowed the acquired samples to be read in groups rather than relying on individual sample polling. The sampling rate obtained from the acquired dataset was approximately 1559 Hz. Therefore, the measured sampling frequency rather than the nominal sensor setting was used for subsequent digital signal processing.

The corresponding Nyquist frequency was calculated as: N=fs2

### 3.2.5 Data Storage and Dataset Preparation

The vibration measurements acquired from the ADXL345 were transmitted to the computer through the ESP32 serial interface and stored as structured comma-separated values (CSV) datasets. Each recorded sample consisted of acceleration measurements corresponding to the three sensor axes, represented as X, Y, and Z. The acquired raw sensor values were subsequently converted into acceleration values in units using the configured sensitivity of the accelerometer. The processed dataset contained the raw sensor readings together with the corresponding calibrated acceleration values, allowing both the original measurements and processed signals to be retained for subsequent analysis. The datasets were organised according to the experimental operating conditions under which the measurements were obtained. This organisation allowed individual datasets to be processed independently and subsequently compared with the baseline dataset. The prepared datasets formed the input for the subsequent signal preprocessing, time-domain analysis, frequency-domain analysis, and feature extraction procedures.

### 3.2.6 Signal Preprocessing

The acquired vibration signals were subjected to preprocessing before feature extraction and frequency-domain analysis. The preprocessing stage was used to obtain consistent and suitable signals for subsequent quantitative analysis. The recorded X, Y, and Z acceleration signals were first examined for data validity and consistency. Sensor offset was corrected using the calibration procedure, and the resulting acceleration signals were expressed in units of. The calibrated signals were then used as the input for subsequent vibration analysis. For dynamic vibration analysis, the mean component of each acceleration signal was removed where required to reduce the influence of the static acceleration component. For a measured acceleration signal, the AC component was obtained as:

arc[n] = x[n] - mean(x)

The same procedure was applied independently to the Y- and Z-axis signals. The resulting signals represented the dynamic vibration component and were used for time-domain and frequency-domain analysis.

The preprocessing procedure was applied consistently to the datasets so that the extracted vibration characteristics could be compared between the baseline and the different experimental conditions.

### 3.2.7 Time-Domain Analysis

The calibrated vibration signals were analysed in the time domain to quantify the magnitude and statistical characteristics of the motor vibration. The analysis was performed independently for the X, Y, and Z acceleration signals.

The principal time-domain features considered were mean, standard deviation, root mean square (RMS), peak value, peak-to-peak value, crest factor, and kurtosis. These parameters provide different measures of vibration magnitude and signal distribution and were used to characterise changes in the vibration response under the experimental conditions investigated. The extracted time-domain features are:

# Chapter Four: Time frame of work

Figure 7 Gantt Chart

# Chapter Five: Expenses

| S.N. | Materials | Cost |
| --- | --- | --- |
| 1 | ESP32 | Rs. 1500 |
| 2 | ADXL345 Accelerometer | Rs. 400 |
| 3 | Breadboard and Jumpers | Rs. 500 |
| 4 | Mounting materials | Rs. 500 |
| 5 | Miscellaneous | Rs. 300 |
|  | Total | Rs. 3700 |

Table 6 Expenses

# Chapter Six: RESULTS AND ANALYSIS

## 6.1 Normal Condition Vibration Analysis.

Ten vibration measurements were acquired under the normal operating condition of the motor. The motor was operated at approximately 1200 rpm during data acquisition. Therefore, the expected rotational frequency of the motor is approximately:  for = 1200/60 = 20Hz. The dominant-frequency analysis of the resultant vibration signal showed a recurring component very close to this expected rotational frequency. Across the ten normal datasets, the frequency corresponding to the running-speed component varied from 19.84 Hz to 20.34 Hz, demonstrating good agreement with the actual operating speed.

| Normal Dataset | Running-speed Frequency (Hz) | Equivalent RPM | Amplitude (g) |
| --- | --- | --- | --- |
| Normal 1 | 19.84 | 1190.5 | 0.1394 |
| Normal 2 | 19.93 | 1195.8 | 0.1211 |
| Normal 3 | 19.97 | 1198.2 | 0.1029 |
| Normal 4 | 20.04 | 1202.2 | 0.0767 |
| Normal 5 | 20.10 | 1206.3 | 0.0731 |
| Normal 6 | 20.14 | 1208.4 | 0.0949 |
| Normal 7 | 20.20 | 1211.8 | 0.1023 |
| Normal 8 | 20.24 | 1214.3 | 0.0816 |
| Normal 9 | 20.28 | 1216.6 | 0.0694 |
| Normal 10 | 20.34 | 1220.3 | 0.1316 |

Table 7 Normal Features1

The mean frequency of this component is approximately 20.10 Hz, corresponding to approximately 1206 rpm. The observed variation is small and is consistent with the actual motor operating speed being around 1200 rpm rather than the nameplate maximum speed of 1420 rpm.

### 6.1.1 Spectral characteristics of the normal condition

The resultant spectrum exhibited several recurring frequency components. In particular, components near 20 Hz, 40 Hz and 80 Hz appeared repeatedly across the ten measurements. The presence of these components in multiple independent normal measurements demonstrates that they form part of the normal spectral signature of the experimental motor and measurement setup.

| Component | Expected Frequency (Hz) | Observed Frequency Range (Hz) | Physical Interpretation |
| --- | --- | --- | --- |
| 1× | ~20 | 19.84–20.34 | Motor rotational frequency |
| 2× | ~40 | 39.68–40.65 | Second harmonic of rotational frequency |
| 4× | ~80 | 79.37–81.32 | Recurring higher harmonic/component |

Table 8 Normal Features 2

The 1× component is particularly significant because it closely follows the actual rotational speed of the motor. The 2× and 4× components occur at approximately integer multiples of the rotational frequency, indicating that the normal vibration signal contains harmonic components associated with the periodic operation of the rotating system.

However, these components should not be individually interpreted as evidence of a fault under the normal condition. Since they occur repeatedly in the baseline measurements, they should instead be considered part of the normal spectral signature against which subsequent operating conditions can be compared.

### 6.1.2 Dominant frequency behaviour in individual axes

The three individual acceleration axes showed different dominant-frequency characteristics. The X-axis displayed the largest variation in dominant frequency. Its highest-ranked frequency moved between approximately 109 Hz, 202–203 Hz, 259–263 Hz and 657–658 Hz across the ten normal measurements. Despite this variation in dominant frequency, the amplitude of the highest-ranked component remained relatively low, between 0.0261 and 0.0449 g. The Y-axis showed considerably greater frequency consistency. Its dominant frequencies remained mainly between approximately 119 and 132 Hz, while amplitudes ranged from 0.0394 to 0.0880 g. The Z-axis also exhibited relatively stable dominant frequencies, mainly around 70–81 Hz, with amplitudes ranging from 0.0511 to 0.0988 g.

| Axis | Dominant Frequency Range (Hz) | Amplitude Range (g) | General Observation |
| --- | --- | --- | --- |
| X | 109.15–658.22 | 0.0261–0.0449 | Highly variable dominant frequency |
| Y | 119.08–132.08 | 0.0394–0.0880 | Relatively stable dominant-frequency region |
| Z | 69.46–81.11 | 0.0511–0.0988 | Relatively stable dominant-frequency region with higher amplitude |

Table 9  Normal Features 3

### 6.1.3 Resultant vibration behavior

The resultant acceleration provides a useful overall representation of the three-axis vibration because it combines the vibration contributions from the X, Y and Z directions. Across the ten normal measurements, the largest resultant spectral peak was generally located around 80 Hz. In Normal 1–4 and Normal 6, the dominant resultant frequency was approximately 79–80 Hz. In Normal 5 and Normal 7–10, the 80 Hz component remained prominent, although the 20 Hz component became the highest-ranked peak in some measurements.

| Normal Dataset | Rank-1 Resultant Frequency (Hz) | Rank-1 Amplitude (g) | Rank of ~20 Hz Component | ~80 Hz Component |
| --- | --- | --- | --- | --- |
| Normal 1 | 79.37 | 0.1469 | 2 | Rank 1 |
| Normal 2 | 79.75 | 0.1855 | 2 | Rank 1 |
| Normal 3 | 79.91 | 0.1185 | 2 | Rank 1 |
| Normal 4 | 80.09 | 0.1562 | 3 | Rank 1 |
| Normal 5 | 20.10 | 0.0731 | 1 | Rank 2 |
| Normal 6 | 80.50 | 0.1054 | 2 | Rank 1 |
| Normal 7 | 20.20 | 0.1023 | 1 | Rank 2 |
| Normal 8 | 81.02 | 0.0851 | 2 | Not in Top 6 |
| Normal 9 | 81.11 | 0.1074 | 3 | Rank 1 |
| Normal 10 | 81.32 | 0.1327 | 5 | Not in Top 6 |

Table 10  Normal Features 4

### 6.1.4 Main findings from normal-condition testing

The normal-condition experiments produced several important baseline findings:

- The actual operating speed was approximately 1200 rpm, corresponding to approximately 20 Hz rotational frequency.

- A clear 1× rotational component was repeatedly detected between 19.84 and 20.34 Hz.

- A 2× component around 40 Hz was repeatedly observed in the resultant spectrum.

- A strong and highly repeatable component around 80 Hz was observed in the resultant vibration. This corresponds approximately to 4× the rotational frequency.

- The resultant 80 Hz component was generally stronger than the 20 Hz component in many normal datasets, particularly Normal 1–4 and Normal 6.

- The Y- and Z-axis dominant-frequency regions were considerably more stable than the X-axis.

- The X-axis showed variable dominant frequencies, including repeated components around 200–270 Hz and approximately 658 Hz. These frequencies should be treated as baseline spectral features rather than fault indicators because they occur during normal operation.

- The normal datasets therefore establish a repeatable baseline spectral signature consisting principally of components around 20 Hz, 40 Hz and 80 Hz in the resultant signal.

- The variation in amplitude between normal measurements demonstrates that vibration amplitude is not perfectly constant even under nominally identical operating conditions. Therefore, a fault-detection system should not rely on a single absolute amplitude value; changes relative to the established normal baseline are more meaningful.

| Finding | Observation |
| --- | --- |
| Actual operating speed | Approximately 1200 rpm |
| Theoretical rotational frequency | Approximately 20 Hz |
| Observed 1× frequency | 19.84–20.34 Hz |
| Observed 2× frequency | 39.68–40.65 Hz |
| Observed 4× frequency | 79.37–81.32 Hz |
| Most repeatable higher-frequency component | Approximately 80 Hz |
| Most stable individual axis | Y and Z compared with X |
| X-axis behavior | Dominant frequency varies considerably between measurements |
| Normal-condition implication | 20, 40 and ~80 Hz components form part of the normal spectral baseline |

Table 11: Normal Findings and Observations

Figure 8 Normal FFT

Figure 9 Normal Autocorelation

Figure 10 Normal CDF

Figure 11 Local Spectral Analysis 1

Figure 12  Local Spectral Analysis 2

Figure 13  Local Spectral Analysis 3

Figure 14 PSD 1

Figure 15 PSD 2

Figure 16 PSD 3

Figure 17 PSD 1 Zoom

Figure 18 PSD 2 Zoom

## 6.2 Analysis of Loaded-Normal Condition

The motor was operated under a normal loaded condition at an actual operating speed of approximately 1200 rpm. Since the motor speed corresponds to approximately 20 Hz, the rotational frequency was taken as the 1× running-speed component. The dominant-frequency results from the two loaded measurements were analyzed for the X, Y, Z axes and resultant vibration.

### 6.2.1 Running-speed component

The most important observation is the presence of the approximately 20 Hz component in the resultant vibration for both loaded measurements.

| Measurement | Frequency (Hz) | Equivalent RPM | Amplitude (g) | Spectral Rank |
| --- | --- | --- | --- | --- |
| Loaded 1 | 19.76 | 1185.88 | 0.1555 | 1 |
| Loaded 2 | 19.70 | 1181.92 | 0.2388 | 1 |

Table 12 Running-speed component of Normally Loaded

The measured frequencies of 19.70–19.76 Hz correspond to approximately 1182–1186 rpm, which is consistent with the actual operating speed of approximately 1200 rpm. This is an important result because the 1× rotational component became the dominant resultant component in both loaded measurements.

### 6.2.2 Comparison with the normal unloaded condition

Under loaded operation, however, the ~20 Hz component became Rank 1 in both measurements due to load suppressing other low-amplitude frequencies.

| Condition | Measurement | Dominant Resultant Frequency (Hz) | Amplitude (g) | Approximate Order |
| --- | --- | --- | --- | --- |
| Normal | 1 | 79.37 | 0.1469 | ~4× |
| Normal | 2 | 79.75 | 0.1855 | ~4× |
| Normal | 3 | 79.91 | 0.1185 | ~4× |
| Normal | 4 | 80.09 | 0.1562 | ~4× |
| Normal | 5 | 20.10 | 0.0731 | ~1× |
| Normal | 6 | 80.50 | 0.1054 | ~4× |
| Normal | 7 | 20.20 | 0.1023 | ~1× |
| Normal | 8 | 81.02 | 0.0851 | ~4× |
| Normal | 9 | 81.11 | 0.1074 | ~4× |
| Normal | 10 | 81.32 | 0.1327 | ~4× |
| Loaded | 1 | 19.76 | 0.1555 | ~1× |
| Loaded | 2 | 19.70 | 0.2388 | ~1× |

Table 13 Comparison

Under unloaded conditions, the resultant spectrum was frequently dominated by a component near 80 Hz, approximately 4× the actual shaft rotational frequency. After applying the normal mechanical load, the dominant resultant component shifted to approximately 20 Hz, corresponding to the shaft rotational frequency. Therefore, loading did not simply increase vibration amplitude; it also changed the relative distribution of spectral energy. The loaded-normal condition exhibited a distinct vibration response compared with the unloaded normal condition. The measured running-speed component remained close to 20 Hz, corresponding to an operating speed of approximately 1182–1186 rpm. In both loaded measurements, the 1× rotational-frequency component was the dominant component of the resultant vibration, with amplitudes of 0.1555 g and 0.2388 g, respectively. A significant increase in the 1× vibration amplitude was observed between the two loaded measurements, with the amplitude increasing by approximately 53.6%. This indicates that the applied mechanical load substantially influences the vibration response at the shaft rotational frequency. In addition to the 1× component, several higher-order spectral components were observed. A particularly repeatable component occurred near 79 Hz in the Z-axis, corresponding approximately to the fourth harmonic of the rotational frequency. Its frequency remained nearly constant between the two measurements, at 79.03 Hz and 78.95 Hz, while its amplitude changed from 0.3303 g to 0.1627 g. The loaded condition also exhibited significant components around 197–198 Hz and 256–257 Hz, particularly in the X-axis. These components represent higher-order spectral content and indicate that the applied load modifies the distribution of vibration energy across the frequency spectrum. Overall, the loaded-normal condition established a characteristic baseline consisting of a strong 1× rotational component in the resultant signal, together with persistent higher-order components, particularly around 4× and approximately 10×–13× the rotational frequency. These characteristics provide a reference for subsequent comparison with mechanically loosened and sudden-load conditions.

Graphs and Plots of Loaded Condition1:

Figure 19 Graphs and Plots of Loaded Condition2:

Graphs and Plots of Loaded Condition2:

## 6.3 Mechanical Looseness

### 6.3.1 Normal baseline vs mechanical looseness: dominant amplitude

| Condition | X Dominant Amplitude (g) | Y Dominant Amplitude (g) | Z Dominant Amplitude (g) | Resultant Dominant Amplitude (g) |
| --- | --- | --- | --- | --- |
| Normal Mean | 0.0390 | 0.0643 | 0.0740 | 0.1213 |
| Lose 6 | 0.1235 | 0.1486 | 0.0708 | 0.1887 |
| Lose 7 | 0.0448 | 0.0178 | 0.0117 | 0.1988 |
| Lose 8 | 0.0340 | 0.0169 | 0.0106 | 0.1824 |

### 6.3.2 Change relative to normal baseline

| Condition | X Change (%) | Y Change (%) | Z Change (%) | Resultant Change (%) |
| --- | --- | --- | --- | --- |
| Lose 6 | +216.7 | +131.0 | -4.3 | +55.6 |
| Lose 7 | +14.9 | +72.3 | +84.2 | +76.3 |
| Lose 8 | +12.8 | +73.7 | +85.7 | +81.5 |

### 6.3.2 Dominant frequencies for mechanical looseness

| Condition | X Frequency (Hz) | X Amplitude (g) | Y Frequency (Hz) | Y Amplitude (g) | Z Frequency (Hz) | Z Amplitude (g) | Resultant Frequency (Hz) | Resultant Amplitude (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lose 6 | 82.05 | 0.1235 | 574.37 | 0.1486 | 779.47 | 0.0708 | 20.53 | 0.1887 |
| Lose 7 | 89.67 | 0.0448 | 107.58 | 0.0178 | 89.67 | 0.0117 | 17.93 | 0.0288 |
| Lose 8 | 82.93 | 0.0340 | 125.64 | 0.0169 | 53.84 | 0.0106 | 11.12 | 0.0224 |

### 6.3.3 Graphs and Plots of Mechanical Looseness:

### 6.3.4 Findings and Discussion – Mechanical Looseness

The vibration characteristics of the motor under mechanical looseness were compared with the normal operating condition using the dominant frequency components obtained from the FFT analysis. Ten measurements were considered for establishing the normal baseline, while Lose 6, Lose 7, and Lose 8 were considered representative measurements of the mechanically loosened condition. Since the motor operated at approximately 1200 rpm during testing, the corresponding rotational frequency was approximately 20 Hz. The normal-condition measurements showed considerable variation in their dominant spectral components. The mean dominant amplitudes of the X, Y, Z and resultant signals were 0.0390 g, 0.0643 g, 0.0740 g and 0.1213 g, respectively. The resultant spectrum frequently contained dominant components around 80 Hz, while some normal measurements showed dominant components close to 20 Hz. Therefore, the presence of a particular frequency component alone was not considered sufficient to identify mechanical looseness. A substantially different response was observed in Lose 6. The dominant X-axis amplitude increased to 0.1235 g, approximately 216.7% higher than the normal mean, while the Y-axis amplitude increased to 0.1486 g, approximately 131.0% higher than the normal mean. The resultant dominant amplitude also increased from the normal mean of 0.1213 g to 0.1887 g, representing an increase of approximately 55.6%. In contrast, the Z-axis dominant amplitude remained close to the normal baseline, changing from 0.0740 g to 0.0708 g. This non-uniform change among the three axes indicates that the vibration response associated with the loosened condition was strongly directional.

The frequency distribution of Lose 6 also differed from the typical normal response. The dominant components occurred at 82.05 Hz in X, 574.37 Hz in Y and 779.47 Hz in Z, while the resultant signal exhibited a dominant component at 20.53 Hz. The 20.53 Hz component is close to the actual rotational frequency of approximately 20 Hz, indicating a strong 1× rotational component in the resultant vibration. At the same time, the high-frequency components observed in the individual axes demonstrate that the loosened mechanical structure generated additional structural vibration components. Lose 7 and Lose 8 exhibited considerably lower dominant amplitudes than Lose 6. For Lose 7, the resultant dominant amplitude was 0.0288 g, while Lose 8 produced 0.0224 g. These values were 76.3% and 81.5% lower, respectively, than the normal mean resultant amplitude. Similar reductions were observed in the Y- and Z-axis dominant amplitudes. However, these lower values should not be interpreted as evidence that looseness was absent, because the dominant-frequency amplitude represents only the largest individual spectral component and does not describe the total vibration energy or the complete spectral distribution. The comparison therefore indicates that mechanical looseness produced a variable vibration response rather than a consistent increase in the amplitude of the single dominant FFT peak. The most pronounced response was observed in Lose 6, characterised by increased X- and Y-direction amplitudes, a strong resultant component close to the running frequency, and additional high-frequency components. The results support the use of multiple vibration features rather than dominant frequency alone for distinguishing mechanical looseness from the normal condition.

6.3.5 Conclusion of Mechanical Looseness Analysis

The mechanical-looseness analysis demonstrated that looseness significantly altered the vibration characteristics of the motor, particularly in terms of vibration amplitude, directional response, and frequency distribution. Comparison with ten normal-condition measurements established a baseline against which the looseness measurements could be evaluated. Among the investigated looseness measurements, loose 6 produced the strongest response, with a resultant dominant amplitude of 0.1887 g compared with a normal baseline mean of 0.1213 g. The X- and Y-axis amplitudes increased to 0.1235 g and 0.1486 g, respectively, compared with normal mean values of 0.0390 g and 0.0643 g. These increases demonstrate that the loosened condition can produce substantial directional amplification of vibration. The resultant dominant frequency of Lose 6 was 20.53 Hz, which closely corresponds to the approximately 20 Hz rotational frequency of the motor at 1200 rpm. In addition, high-frequency dominant components were observed at 574.37 Hz in the Y-axis and 779.47 Hz in the Z-axis. These components indicate that the mechanical looseness affected not only the fundamental rotational response but also the dynamic behaviour of the motor-support structure. However, Lose 7 and Lose 8 did not exhibit the same increase in dominant amplitude. Their resultant dominant amplitudes were 0.0288 g and 0.0224 g, respectively. This variation demonstrates that mechanical looseness cannot be reliably identified using only the amplitude of the single highest FFT peak. The vibration response is influenced by the severity and dynamic state of the loosened assembly, as well as the direction of measurement.

Overall, the results establish that mechanical looseness produces measurable changes in the vibration signature, particularly through directional amplitude changes, the presence of the running-frequency component, and changes in the distribution of higher-frequency spectral components. Consequently, the findings support the use of combined time-domain statistical features and frequency-domain features for the subsequent fault-classification stage of the proposed vibration-based condition-monitoring system.

## 6.4 Sudden Load

### 6.4.1 Time-domain features

| Condition | RMS (g) | Absolute Peak (g) | Peak-to-Peak (g) | Crest Factor | Impulse Factor | Clearance Factor |
| --- | --- | --- | --- | --- | --- | --- |
| Normal baseline means | 0.6120 | 3.5835 | 3.5757 | 5.9115 | 7.6005 | 8.6530 |
| Sudden Load 1 | 0.5215 | 3.5363 | 3.5257 | 6.7805 | 8.3493 | 9.2603 |
| Sudden Load 2 | 0.4536 | 3.2335 | 3.2269 | 7.1285 | 8.4801 | 9.2434 |

### 6.4.2 Statistical Features

| Condition | Mean (g) | Std. Dev. (g) | Variance | Skewness | Kurtosis | Energy | Signal Power |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal baseline means | 0.4763 | 0.3840 | 0.1470 | 2.3787 | 9.72* | 18904.8 | 0.3780 |
| Sudden Load 1 | 0.4235 | 0.3043 | 0.0926 | 2.3688 | 9.3839 | 13583 | 0.2720 |
| Sudden Load 2 | 0.3813 | 0.2457 | 0.0604 | 2.2408 | 8.9479 | 10274 | 0.2058 |

### 6.4.3 Frequency Domain Features

| Condition | Dominant Amplitude (g) | Spectral Centroid (Hz) | Spectral RMS Frequency (Hz) | Bandwidth (Hz) | Spectral Energy | Spectral Variance |
| --- | --- | --- | --- | --- | --- | --- |
| Normal baseline means | 0.4759 | 119.47 | 216.90 | 181.02 | 0.6890 | 32792 |
| Sudden Load 1 | 0.4371 | 106.68 | 213.61 | 185.06 | 0.4918 | 34248 |
| Sudden Load 2 | 0.4092 | 114.93 | 222.38 | 190.38 | 0.3913 | 36243 |

### 6.4.4 Spectral Features

| Condition | Spectral Skewness | Spectral Kurtosis | Spectral Flatness | Spectral Entropy | Spectral Rolloff (Hz) | Significant Peaks |
| --- | --- | --- | --- | --- | --- | --- |
| Normal baseline means | 1.9772 | 6.2381 | 0.1798 | 0.6027 | 250.06 | 5 |
| Sudden Load 1 | 1.9369 | 5.9172 | 0.1589 | 0.5335 | 272.81 | 4 |
| Sudden Load 2 | 1.8158 | 5.4099 | 0.1569 | 0.5233 | 311.63 | 5 |

### 6.4.5 Findings

- The most significant findings were:

- Overall RMS vibration decreased under both sudden-load measurements.

- Despite the lower RMS, crest factor increased from approximately 5.91 under normal operation to 6.78 and 7.13.

- Impulse factor and clearance factor also increased, indicating relatively stronger individual vibration excursions compared with the overall vibration level.

- Total signal energy decreased by approximately 28% in Sudden Load 1 and 46% in Sudden Load 2.

- Spectral bandwidth increased, indicating a somewhat broader frequency distribution.

- Spectral Rolloff increased, particularly in Sudden Load 2, indicating greater extension of spectral content toward higher frequencies.

- Spectral entropy and flatness decreased, suggesting a more concentrated and structured vibration spectrum.

- The number of significant spectral peaks remained approximately unchanged, with four peaks in Sudden Load 1 and five in Sudden Load 2 compared with five in the normal baseline.

- The dominant-frequency value of approximately 0.031 Hz in the aggregate feature table should not be interpreted as the actual motor rotational frequency. The physical running frequency should be assessed from the detailed FFT peak tables around the approximately 20 Hz shaft frequency.

### 6.4.6 Conclusion

The sudden-load experiments demonstrated that an abrupt change in loading condition produced measurable changes in both the time-domain and frequency-domain characteristics of the motor vibration signal. Unlike a condition characterised by a simple increase in vibration magnitude, the sudden-load response exhibited lower RMS vibration and spectral energy while simultaneously producing higher crest, impulse, and clearance factors. This indicates that the loading event altered the relative distribution of vibration amplitudes and increased the prominence of individual vibration excursions.

The frequency-domain analysis further showed a moderate broadening of the spectrum, increased spectral Rolloff and reduced spectral entropy and flatness. These changes indicate that the sudden-load condition modified the spectral distribution and produced a more structured vibration response. Sudden Load 2 showed the strongest reduction in overall energy and the largest increase in spectral Rolloff, suggesting a more pronounced change in the distribution of frequency components.

Overall, the results demonstrate that sudden loading cannot be reliably identified using RMS or dominant amplitude alone. The combination of crest factor, impulse factor, clearance factor, spectral bandwidth, spectral Rolloff, entropy and spectral energy provides a more informative representation of the vibration response to an abrupt loading event. The results therefore support the use of multiple complementary vibration features for distinguishing transient loading conditions from steady normal operation.

### 6.4.7 Plots and Graph

## 6.5 Load Imbalance

### 6.5.1 Baseline for Load Imbalance

Normal-condition time-domain baseline

| Feature | X Mean | X SD | Y Mean | Y SD | Z Mean | Z SD | Resultant Mean | Resultant SD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMS (g) | 0.1777 | 0.0510 | 0.5433 | 0.0647 | 0.2736 | 0.0349 | 0.3793 | 0.0364 |
| Absolute Peak (g) | 1.2578 | 0.2052 | 2.8944 | 0.0060 | 1.9828 | 0.0300 | 2.9716 | 0.2453 |
| Peak-to-Peak (g) | 2.3099 | 0.3450 | 3.9961 | 0.0000 | 3.6042 | 0.1420 | 3.4679 | 0.1832 |
| Crest Factor | 7.6328 | 2.9080 | 5.3825 | 0.6969 | 7.3290 | 0.9520 | 7.8867 | 1.0636 |
| Shape Factor | 1.2941 | 0.0625 | 1.3921 | 0.0641 | 1.3869 | 0.0321 | 1.3987 | 0.0384 |
| Impulse Factor | 9.9602 | 3.9077 | 7.5144 | 1.2112 | 10.1745 | 1.2761 | 11.0586 | 1.6834 |
| Clearance Factor | 11.9291 | 4.7283 | 9.4104 | 1.8130 | 12.5798 | 1.7970 | 13.3152 | 2.1941 |
| Skewness | 0.0657 | 0.0394 | -0.7881 | 0.1220 | -0.3092 | 0.2422 | 2.1924 | 0.3395 |
| Kurtosis | 1.4778 | 1.4504 | 2.9463 | 1.3705 | 3.3003 | 1.1955 | 7.7940 | 2.3665 |
| Energy | 1664.71 | 972.58 | 14893.33 | 3387.87 | 3780.52 | 917.24 | 7233.92 | 1395.16 |
| Signal Power | 0.0333 | 0.0195 | 0.2980 | 0.0680 | 0.0756 | 0.0184 | 0.1447 | 0.0278 |
| IQR (g) | 0.2266 | 0.0887 | 0.5573 | 0.1289 | 0.2891 | 0.0488 | 0.3796 | 0.0586 |
| Zero Crossing Rate | 0.5030 | 0.0421 | 0.3702 | 0.1220 | 0.4145 | 0.0992 | 0.2945 | 0.0706 |

Main baseline vibration intensity

| Axis | Mean RMS (g) | SD (g) | Relative contribution |
| --- | --- | --- | --- |
| X | 0.1777 | 0.0510 | Lowest |
| Y | 0.5433 | 0.0647 | Highest |
| Z | 0.2736 | 0.0349 | Intermediate |
| Resultant | 0.3793 | 0.0364 | Overall vibration |

Peak and impulsive behavior

| Feature | X Mean | Y Mean | Z Mean | Resultant Mean |
| --- | --- | --- | --- | --- |
| Absolute Peak (g) | 1.2578 | 2.8944 | 1.9828 | 2.9716 |
| Peak-to-Peak (g) | 2.3099 | 3.9961 | 3.6042 | 3.4679 |
| Crest Factor | 7.6328 | 5.3825 | 7.3290 | 7.8867 |
| Impulse Factor | 9.9602 | 7.5144 | 10.1745 | 11.0586 |
| Clearance Factor | 11.9291 | 9.4104 | 12.5798 | 13.3152 |

Statistical distribution baseline

| Feature | X Mean | Y Mean | Z Mean | Resultant Mean |
| --- | --- | --- | --- | --- |
| Skewness | 0.0657 | -0.7881 | -0.3092 | 2.1924 |
| Kurtosis | 1.4778 | 2.9463 | 3.3003 | 7.7940 |

Frequency-domain baseline

| Feature | X Mean | X SD | Y Mean | Y SD | Z Mean | Z SD | Resultant Mean | Resultant SD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dominant Frequency (Hz) | 360.31 | 262.14 | 121.02 | 1.40 | 73.88 | 10.89 | 53.77 | 30.61 |
| Dominant Amplitude (g) | 0.0473 | 0.0321 | 0.0624 | 0.0277 | 0.0632 | 0.0109 | 0.1043 | 0.0272 |
| Spectral Centroid (Hz) | 422.90 | 22.69 | 295.85 | 32.50 | 354.53 | 27.47 | 322.50 | 39.47 |
| Spectral RMS Frequency (Hz) | 489.91 | 44.84 | 308.52 | 90.35 | 433.30 | 65.70 | 295.58 | 61.02 |
| Spectral Bandwidth (Hz) | 256.33 | 22.73 | 223.98 | 19.54 | 279.90 | 8.80 | 239.48 | 13.68 |
| Spectral Energy | 0.0971 | 0.0570 | 0.8517 | 0.1347 | 0.2170 | 0.0430 | 0.4217 | 0.0350 |
| Spectral Variance | 66050 | 11936 | 50420 | 8972 | 78397 | 4962 | 57473 | 6613 |
| Spectral Flatness | 0.7117 | 0.0697 | 0.6926 | 0.0039 | 0.6873 | 0.0098 | 0.7610 | 0.0227 |
| Spectral Entropy | 0.8055 | 0.0910 | 0.8709 | 0.0251 | 0.8405 | 0.0180 | 0.8135 | 0.0181 |
| Spectral Rolloff (Hz) | 727.95 | 31.34 | 437.58 | 143.08 | 738.91 | 26.50 | 450.20 | 141.23 |
| Significant Peaks | 1948 | 1275 | 3812 | 1026 | 1892 | 276 | 1698 | 642 |

### 6.5.2 Normal-condition baseline characterization

Three independent measurements were obtained under normal operating conditions using the second motor to establish a motor-specific vibration baseline. The vibration signals were processed after removal of the DC component, and frequency-domain analysis was performed using a Hann-windowed FFT. The sampling frequency was 1559 Hz. The extracted results showed a clear directional dependence of the motor vibration. The Y-axis exhibited the highest mean RMS acceleration of 0.5433 g, followed by the Z-axis at 0.2736 g and the X-axis at 0.1777 g. The resultant RMS acceleration was 0.3793 ± 0.0364 g, indicating comparatively stable overall vibration magnitude across the three normal measurements.

Peak-related features showed that the motor naturally exhibited relatively strong transient components during normal operation. The mean resultant absolute peak and peak-to-peak acceleration were 2.9716 g and 3.4679 g, respectively, while the resultant crest factor averaged 7.8867. The resultant kurtosis was 7.7940, indicating a comparatively peaked and heavy-tailed acceleration distribution. Consequently, these features were retained as baseline indicators rather than being interpreted individually as evidence of a fault."

Frequency-domain analysis showed that the normal vibration spectrum was distributed across a relatively broad frequency range. The dominant frequency was not consistent between the three normal measurements, particularly in the X-axis, indicating that the single highest FFT peak was not sufficiently stable to serve as the sole diagnostic indicator. In contrast, the low-frequency spectra showed more useful recurring behavior. Strong components were observed predominantly in the approximately 60–80 Hz region in the Y and Z directions, while the resultant low-frequency peak amplitudes ranged from approximately 0.073 to 0.121 g.

The normal-condition results therefore establish a reference vibration signature characterized by comparatively high Y-axis RMS vibration, intermediate Z-axis vibration, lower but more variable X-axis vibration, and strong low-frequency spectral components in the Y and Z directions. Subsequent load-imbalance measurements should be evaluated against these baseline ranges, with particular attention given to changes in resultant RMS, directional RMS distribution, low-frequency spectral amplitude, and the emergence or strengthening of repeatable rotational-frequency-related components.

### 6.5.3 Load Imbalance Condition Analysis

#### 6.5.3.1 Time-Domain Comparison

| Condition | Dataset | Description |
| --- | --- | --- |
| Normal / Baseline | Normal | Motor operating without intentional imbalance |
| Load Imbalance 1 | Sample 1 | Motor operated with induced load imbalance |
| Load Imbalance 2 | Sample 2 | Motor operated with induced load imbalance |
| Load Imbalance 3 | Sample 3 | Motor operated with induced load imbalance |
| Load Imbalance 4 | Sample 4 | Motor operated with induced load imbalance |

#### 6.5.3.2 RMS vibration level for baseline and load-imbalance samples

| Condition | X RMS (g) | Y RMS (g) | Z RMS (g) |
| --- | --- | --- | --- |
| Normal / Baseline | 1.3128 | 1.6723 | 1.8035 |
| Load Imbalance 1 | 1.5213 | 1.7402 | 1.8135 |
| Load Imbalance 2 | 1.5213 | 1.7402 | 1.8135 |
| Load Imbalance 3 | 1.4983 | 1.7688 | 1.8222 |
| Load Imbalance 4 | 1.4452 | 1.7696 | 1.8251 |

Table 14 RMS vibration level for baseline and load-imbalance samples

#### 6.5.3.3 Change in RMS vibration relative to the normal baseline

| Condition | X RMS Change (%) | Y RMS Change (%) | Z RMS Change (%) |
| --- | --- | --- | --- |
| Normal / Baseline | 0.00 | 0.00 | 0.00 |
| Load Imbalance 1 | +15.89 | +4.06 | +0.56 |
| Load Imbalance 2 | +15.89 | +4.06 | +0.56 |
| Load Imbalance 3 | +14.14 | +5.77 | +1.04 |
| Load Imbalance 4 | +10.08 | +5.82 | +1.20 |

Table 15 Change in RMS vibration relative to the normal baseline

6.5.3.4 Crest Factor Comparison

| Condition | X Peak (g) | Y Peak (g) | Z Peak (g) |
| --- | --- | --- | --- |
| Normal / Baseline | 2.0818 | 2.3479 | 2.0412 |
| Load Imbalance 1 | 2.0477 | 2.2744 | 2.1083 |
| Load Imbalance 2 | 2.0477 | 2.2744 | 2.1083 |
| Load Imbalance 3 | 2.0668 | 2.2090 | 2.0924 |
| Load Imbalance 4 | 2.1109 | 2.1550 | 2.0487 |

Table 16 Crest Factor Comparison

Dominant frequency and dominant amplitude

Table 17 Dominant frequency and dominant amplitude

| Condition | X Dominant Frequency (Hz) | X Amplitude (g) | Y Dominant Frequency (Hz) | Y Amplitude (g) | Z Dominant Frequency (Hz) | Z Amplitude (g) |
| --- | --- | --- | --- | --- | --- | --- |
| Normal / Baseline | 49.063 | 1.6136 | 49.063 | 2.1713 | 49.063 | 2.3364 |
| Load Imbalance 1 | 49.060 | 1.9432 | 49.060 | 2.2910 | 49.060 | 2.3809 |
| Load Imbalance 2 | 49.060 | 1.9432 | 49.060 | 2.2910 | 49.060 | 2.3809 |
| Load Imbalance 3 | 49.021 | 1.7999 | 49.021 | 2.2203 | 49.021 | 2.2803 |
| Load Imbalance 4 | 48.981 | 1.7593 | 48.981 | 2.3302 | 48.981 | 2.3856 |

Dominant-frequency amplitude change relative to baseline

| Condition | X Change (%) | Y Change (%) | Z Change (%) |
| --- | --- | --- | --- |
| Normal / Baseline | 0.00 | 0.00 | 0.00 |
| Load Imbalance 1 | +20.44 | +5.52 | +1.90 |
| Load Imbalance 2 | +20.44 | +5.52 | +1.90 |
| Load Imbalance 3 | +11.52 | +2.26 | -2.40 |
| Load Imbalance 4 | +9.01 | +7.32 | +2.11 |

Table 18 Dominant-frequency amplitude change relative to baseline

Dominant-frequency equivalent rotational frequency

| Condition | Dominant Frequency (Hz) | Equivalent RPM (frequency × 60) |
| --- | --- | --- |
| Normal / Baseline | 49.063 | 2943.8 |
| Load Imbalance 1 | 49.060 | 2943.6 |
| Load Imbalance 2 | 49.060 | 2943.6 |
| Load Imbalance 3 | 49.021 | 2941.2 |
| Load Imbalance 4 | 48.981 | 2938.8 |

Overall comparison and principal findings

| Feature | Normal Baseline | Load Imbalance Behaviour | Diagnostic Observation |
| --- | --- | --- | --- |
| X-axis RMS | 1.3128 g | 1.445–1.521 g | Clear increase |
| Y-axis RMS | 1.6723 g | 1.740–1.770 g | Moderate increase |
| Z-axis RMS | 1.8035 g | 1.814–1.825 g | Small increase |
| X-axis dominant amplitude | 1.6136 g | 1.759–1.943 g | Strong increase |
| Y-axis dominant amplitude | 2.1713 g | 2.220–2.330 g | Moderate increase |
| Z-axis dominant amplitude | 2.3364 g | 2.280–2.386 g | Small variation |
| Dominant frequency | ~49.06 Hz | ~48.98–49.06 Hz | Frequency remains stable |
| 2× component | Weak | Becomes more pronounced | Harmonic content increases |
| Spectral energy | Baseline level | Generally higher | Increased vibration energy |
| Crest factor | Relatively stable | No consistent increase | Not primary indicator |

Table 19: Overall comparison and principal findings

The load-imbalance condition produced a substantial increase in the measured vibration response compared with the normal baseline condition. The average RMS acceleration increased from approximately 0.713, 0.213 and 0.291 g in the X, Y and Z axes under normal operation to 1.444, 1.738 and 1.816 g under load imbalance, corresponding to increases of approximately 103%, 714% and 524%, respectively. The Z-axis exhibited the highest and most consistent vibration level.

A major change was also observed in the frequency domain. Under normal operation, the dominant spectral component occurred near 100.2 Hz, whereas all four load-imbalance samples exhibited a highly repeatable dominant component near 49.0 Hz in all three axes. The approximately 49 Hz component remained within a very narrow frequency range across the four measurements, demonstrating good repeatability.

The amplitude of the approximately 49 Hz component increased from only about 0.04–0.05 g in the normal baseline to approximately 1.78 g, 2.25 g and 2.35 g in the X, Y and Z axes, respectively, under load imbalance. This represents an amplification of approximately 43–84 times depending on the measurement direction. Additional components near 98 Hz and 147 Hz were also observed, indicating the presence of harmonic vibration associated with the dominant periodic response.

The results therefore demonstrate a clear and repeatable change in both vibration magnitude and spectral characteristics when load imbalance was introduced. The combination of increased RMS acceleration, strong amplification of the approximately 49 Hz component, and the appearance of associated harmonic components provides a distinct vibration signature for the experimentally imposed load-imbalance condition.

The most significant change was the large increase in vibration amplitude together with the emergence of a strong and highly repeatable approximately 49 Hz spectral component. The Z-axis showed the highest vibration response, reaching approximately 1.83 g RMS and 2.39 g dominant spectral amplitude in the tested samples. The approximately 49 Hz component was accompanied by higher-frequency harmonic components near 98 Hz and 147 Hz.

The consistency of the approximately 49 Hz component across all four load-imbalance samples, together with the large increase in RMS and spectral energy relative to the normal baseline, indicates that the measured vibration response was strongly affected by the introduced imbalance. These results demonstrate that dominant-frequency amplitude, RMS vibration level and harmonic spectral structure can provide effective features for distinguishing the experimentally imposed load-imbalance condition from normal operation.

# Chapter Seven: Conclusion

| Condition | Experimental condition | Primary vibration signature | Key frequency characteristics | Amplitude behavior | Most informative axis/component | Physical significance | Diagnostic interpretation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | Motor operating without intentionally introduced abnormality | Stable and repeatable baseline vibration | ~20 Hz (1×), ~40 Hz (2×), recurring ~80 Hz component | Relatively stable | Resultant and Z-axis | Represents inherent vibration of the motor, mounting and rotating system | Reference condition |
| Normally Loaded | Motor coupled to generator with normal applied load | Increased and redistributed vibration energy | Strong ~20 Hz; persistent ~79 Hz; additional components around ~197–198 Hz and ~256–257 Hz | Increased resultant vibration compared with normal operation | Resultant, Z-axis and X-axis | Loading changes shaft/load interaction and vibration transmission | Load-related spectral change |
| Sudden Load | Rapid application/change of generator load | Transient vibration response | Temporary changes in spectral content during load application | Rapid amplitude variation | Resultant and axes showing strongest transient response | Rapid torque/load variation produces a transient mechanical response | Characteristic of changing operating condition |
| Mechanical Looseness | Intentionally introduced mechanical looseness | Complex and variable spectral pattern | Multiple altered and higher-frequency components | Increased and variable vibration response | X, Y, Z and resultant | Loose interfaces permit additional relative motion and structural response | Consistent with mechanical looseness |
| Load Imbalance | Intentional rotor/load imbalance | Increased rotational vibration | Strong emphasis on running-speed (1×) component | Increased vibration relative to normal baseline | Resultant and radial-sensitive axes | Eccentric mass generates centrifugal force at rotational frequency | Consistent with load imbalance |

Table 20 Conclusion

# References

1) Hsia, S. C., & Hsu, C. Y. (2024). Real-Time Monitor and Control System for Abnormal Motor Vibrations. IEEE Transactions on Instrumentation and Measurement, 73, 1–11. https://doi.org/10.1109/TIM.2023.3331405

2) Joung, B. G., Lee, W. J., Huang, A., & Sutherland, J. W. (2020). Development and application of a method for real time motor fault detection. Procedia Manufacturing, 49, 94–98. https://doi.org/10.1016/j.promfg.2020.07.002

3) Magadán, L., Suárez, F. J., Granda, J. C., & García, D. F. (2020). Low-cost real-time monitoring of electric motors for the industry 4.0. Procedia Manufacturing, 42, 393–398. https://doi.org/10.1016/j.promfg.2020.02.057

4) Quan, Y., Lin, F., Luo, D., & Xue, W. (2025). Design of Embedded Motor Diagnostic System Based on Vibration Signal Analysis and Machine Learning. ICTCE 2024 - Proceedings of the 2024 6th International Conference on Telecommunications and Communication Engineering, 96–100. https://doi.org/10.1145/3705391.3705407

5) Ribeiro Junior, R. F., Areias, I. A. dos S., & Gomes, G. F. (2021). Fault detection and diagnosis using vibration signal analysis in frequency domain for electric motors considering different real fault types. Sensor Review, 41, 311–319. https://doi.org/10.1108/SR-02-2021-0052

6) ADXL345 Datasheet and Product Info | Analog Devices

7) ESP32 Datasheet | Espresso Documentation

8) NumPy Documentation

9) Hann — SciPy v1.18.0 Manual

10) SciPy documentation — SciPy v1.18.0 Manual

# APPENDIX I

Code Used in MCU

#include <Sigh>

// ============================================================
// ESP32 ↔ ADXL345 PINS
// ============================================================

#define CS_PIN    5
#define SCK_PIN   18
#define MISO_PIN  19
#define MOSI_PIN  23
#define INT1_PIN  27

// ============================================================
// ADXL345 REGISTERS
// ============================================================

#define DEVID        0x00
#define BW_RATE      0x2C
#define POWER_CTL    0x2D
#define INT_ENABLE   0x2E
#define INT_MAP      0x2F
#define INT_SOURCE   0x30
#define DATA_FORMAT  0x31
#define DATAX0       0x32
#define FIFO_CTL     0x38
#define FIFO_STATUS  0x39

// ============================================================
// CONFIGURATION
// ============================================================

#define BAUD_RATE 921600

// ADXL345 nominal 1600 Hz
// Actual rate measured previously ≈ 1559.9 Hz

#define FIFO_WATERMARK 8

Spica’s sip(VSPI);

// ============================================================
// INTERRUPT FLAG
// ============================================================

volatile bool fifoWatermark = false;

void IRAM_ATTR fifoISR()
{
  fifoWatermark = true;
}

// ============================================================
// SPI WRITE
// ============================================================

void writeRegister(uint8_t reg, uint8_t value)
{
  spi.beginTransaction(
    SPISettings(8000000, MSBFIRST, SPI_MODE3)
  );

  digitalWrite(CS_PIN, LOW);

  spi.transfer(reg & 0x3F);
  spi.transfer(value);

  digitalWrite(CS_PIN, HIGH);

  spi.endTransaction();
}

// ============================================================
// SPI READ REGISTER
// ============================================================

uint8_t readRegister(uint8_t reg)
{
  spi.beginTransaction(
    SPISettings(8000000, MSBFIRST, SPI_MODE3)
  );

  digitalWrite(CS_PIN, LOW);

  spi.transfer(0x80 | reg);

  uint8_t value = spi.transfer(0x00);

  digitalWrite(CS_PIN, HIGH);

  spi.endTransaction();

  return value;
}

// ============================================================
// READ ONE XYZ SAMPLE
// ============================================================

void readXYZ(
  int16_t &x,
  int16_t &y,
  int16_t &z
)
{
  uint8_t d[6];

  spi.beginTransaction(
    SPISettings(8000000, MSBFIRST, SPI_MODE3)
  );

  digitalWrite(CS_PIN, LOW);

  // 0xC0 = READ + MULTI-BYTE
  spi.transfer(0xC0 | DATAX0);

  for (int i = 0; i < 6; i++)
  {
    d[i] = spi.transfer(0x00);
  }

  digitalWrite(CS_PIN, HIGH);

  spi.endTransaction();

  x = (int16_t)(
    ((uint16_t)d[1] << 8) | d[0]
  );

  y = (int16_t)(
    ((uint16_t)d[3] << 8) | d[2]
  );

  z = (int16_t)(
    ((uint16_t)d[5] << 8) | d[4]
  );
}

// ============================================================
// DRAIN FIFO
// ============================================================

void drainFIFO()
{
  uint8_t status = readRegister(FIFO_STATUS);

  uint8_t entries = status & 0x3F;

  while (entries > 0)
  {
    int16_t x;
    int16_t y;
    int16_t z;

    readXYZ(x, y, z);

    // ========================================================
    // RAW DATA OUTPUT
    // ========================================================

    Serial.print(x);
    Serial.print(',');
    Serial.print(y);
    Serial.print(',');
    Serial.println(z);

    entries--;
  }
}

// ============================================================
// SETUP
// ============================================================

void setup()
{
  Serial.begin(BAUD_RATE);

  pinMode(CS_PIN, OUTPUT);
  digitalWrite(CS_PIN, HIGH);

  pinMode(INT1_PIN, INPUT);

  spi.begin(
    SCK_PIN,
    MISO_PIN,
    MOSI_PIN,
    CS_PIN
  );

  delay(500);

  // ==========================================================
  // VERIFY ADXL345
  // ==========================================================

  uint8_t id = readRegister(DEVID);

  if (id != 0xE5)
  {
    // Don't print anything except an obvious error.
    Serial.println("ERROR_ADXL345");

    while (1)
    {
      delay(1000);
    }
  }

  // ==========================================================
  // STANDBY
  // ==========================================================

  writeRegister(
    POWER_CTL,
    0x00
  );

  // ==========================================================
  // ±2g FULL RESOLUTION
  // ==========================================================

  writeRegister(
    DATA_FORMAT,
    0x08
  );

  // ==========================================================
  // 1600 Hz NOMINAL ODR
  // ==========================================================

  writeRegister(
    BW_RATE,
    0x0E
  );

  // ==========================================================
  // FIFO STREAM MODE
  //
  // 0x80 = STREAM
  // 0x08 = watermark 8
  //
  // 0x88 = STREAM + WATERMARK 8
  // ==========================================================

  writeRegister(
    FIFO_CTL,
    0x88
  );

  // ==========================================================
  // WATERMARK → INT1
  // ==========================================================

  writeRegister(
    INT_MAP,
    0x00
  );

  // ==========================================================
  // ENABLE WATERMARK INTERRUPT ONLY
  //
  // WATERMARK = bit 1
  // ==========================================================

  writeRegister(
    INT_ENABLE,
    0x02
  );

  // ==========================================================
  // START MEASUREMENT
  // ==========================================================

  writeRegister(
    POWER_CTL,
    0x08
  );

  delay(100);

  // Clear any pending interrupt
  readRegister(INT_SOURCE);

  // ==========================================================
  // ATTACH GPIO27 INTERRUPT
  // ==========================================================

  attachInterrupt(
    digitalPinToInterrupt(INT1_PIN),
    fifoISR,
    RISING
  );

  // ==========================================================
  // CSV HEADER
  // ==========================================================

  Serial.println("X,Y,Z");
}

// ============================================================
// LOOP
// ============================================================

void loop()
{
  if (fifoWatermark)
  {
    fifoWatermark = false;

    drainFIFO();
  }

  // Safety:
  // If the interrupt flag somehow gets missed,
  // check FIFO occasionally.
  //
  // We don't continuously poll it because we want
  // minimal SPI traffic.
  static uint32_t lastSafetyCheck = 0;

  if (micros() - lastSafetyCheck >= 2000)
  {
    lastSafetyCheck = micros();

    uint8_t status =
      readRegister(FIFO_STATUS);

    uint8_t entries =
      status & 0x3F;

    if (entries >= 16)
    {
      drainFIFO();
    }
  }
}

Python Based Data Acquisition Code

import serial
import csv
import time
import os



PORT = "COM6"          
BAUDRATE = 921600
DURATION = 30.0

OUTPUT_FILE = "data/raw/adxl345_30sec_raw2.csv"



print("Connecting to ESP32...")

ser = serial.Serial(
    PORT,
    BAUDRATE,
    timeout=1
)

time.sleep(2)
ser.reset_input_buffer()

print("Connected.")



print("Waiting for valid XYZ data...")

while True:
    line = ser.readline().decode(errors="ignore").strip()

    if not line:
        continue

    parts = line.split(",")

    if len(parts) == 3:
        try:
            x = int(parts[0])
            y = int(parts[1])
            z = int(parts[2])

            print("Valid sensor data detected.")
            break

        except ValueError:
            pass



print("\nAcquisition starts in 2 seconds...")
time.sleep(2)

# Tell ESP32 to start if your firmware expects a command
ser.write(b"START\n")

time.sleep(0.1)



print("\n========================================")
print("30-SECOND ACQUISITION STARTED")
print("========================================")

samples = []

start_time = time.perf_counter()
last_report = start_time

while True:

    now = time.perf_counter()

    if now - start_time >= DURATION:
        break

    line = ser.readline().decode(errors="ignore").strip()

    if not line:
        continue

    parts = line.split(",")

    if len(parts) != 3:
        continue

    try:
        x = int(parts[0])
        y = int(parts[1])
        z = int(parts[2])

        samples.append((x, y, z))

    except ValueError:
        continue

    # Progress every second
    if now - last_report >= 1.0:

        elapsed = now - start_time
        rate = len(samples) / elapsed

        print(
            f"Elapsed: {elapsed:5.1f} s | "
            f"Samples: {len(samples):6d} | "
            f"Rate: {rate:7.2f} Hz"
        )

        last_report = now



end_time = time.perf_counter()

actual_duration = end_time - start_time
sample_count = len(samples)
effective_rate = sample_count / actual_duration

os.makedirs(os.path.dirname(OUTPUT_FILE), exist_ok=True)

with open(OUTPUT_FILE, "w", newline="") as f:

    writer = csv.writer(f)

    writer.writerow(["X", "Y", "Z"])

    writer.writerows(samples)



print("\n========================================")
print("ACQUISITION COMPLETE")
print("========================================")

print(f"Duration:       {actual_duration:.6f} s")
print(f"Samples:        {sample_count}")
print(f"Effective rate: {effective_rate:.3f} Hz")

print("\nFirst 10 samples:")

for sample in samples[:10]:
    print(sample)

print("\nCSV:")
print(os.path.abspath(OUTPUT_FILE))

print("========================================")

ser.close()

Python Code to remove static noice and gravity component from acquired vibration data.

import pandas as pd
import numpy as np
from pathlib import Path



INPUT_FILE = Path("datanormalAC5/raw/adxl345_30sec_raw2.csv")
OUTPUT_FILE = Path("datanormalAC5/processed/adxl345_5sec_calibrated.csv")

OUTPUT_FILE.parent.mkdir(parents=True, exist_ok=True)



X_OFFSET = 0.0411
Y_OFFSET = 0.9148
Z_OFFSET = -0.0110




COUNTS_PER_G = 256.0




print("Loading:", INPUT_FILE)

df = pd.read_csv(INPUT_FILE)

print(f"Raw samples loaded: {len(df)}")




df["X_g_raw"] = df["X"] / COUNTS_PER_G
df["Y_g_raw"] = df["Y"] / COUNTS_PER_G
df["Z_g_raw"] = df["Z"] / COUNTS_PER_G




df["X_g"] = df["X_g_raw"] - X_OFFSET
df["Y_g"] = df["Y_g_raw"] - Y_OFFSET
df["Z_g"] = df["Z_g_raw"] - Z_OFFSET




df.to_csv(
    OUTPUT_FILE,
    index=False
)



print()
print("========================================")
print("CALIBRATION COMPLETE")
print("========================================")

print(f"Samples: {len(df)}")

print()
print("Mean BEFORE calibration:")

print(
    f"X = {df['X_g_raw'].mean():.6f} g"
)

print(
    f"Y = {df['Y_g_raw'].mean():.6f} g"
)

print(
    f"Z = {df['Z_g_raw'].mean():.6f} g"
)

print()
print("Mean AFTER calibration:")

print(
    f"X = {df['X_g'].mean():.6f} g"
)

print(
    f"Y = {df['Y_g'].mean():.6f} g"
)

print(
    f"Z = {df['Z_g'].mean():.6f} g"
)

print()
print("Saved:")
print(OUTPUT_FILE.resolve())

print("========================================")

Feature Extraction Code( RMS PEAK FFR SPD )
#!/usr/bin/env python3


import os
import sys
import math
import warnings
from pathlib import Path

import numpy as np
import pandas as pd
from scipy import stats
from scipy.signal import find_peaks, peak_prominences, welch, spectrogram as sp_spectrogram
from scipy.fft import rfft, rfftfreq

import matplotlib
matplotlib.use("Agg")
import matplotlib.pyplot as plt
import matplotlib.ticker as mticker

from reportlab.lib.pagesizes import A4
from reportlab.lib.units import mm
from reportlab.lib import colors
from reportlab.lib.styles import getSampleStyleSheet, ParagraphStyle
from reportlab.lib.enums import TA_CENTER, TA_LEFT
from reportlab.platypus import (
    SimpleDocTemplate, Paragraph, Spacer, Table, TableStyle, Image,
    PageBreak, KeepTogether, HRFlowable
)
from reportlab.platypus.flowables import Flowable

warnings.filterwarnings("ignore", category=RuntimeWarning)



BASE_DIR = Path(__file__).resolve().parent

# ---- Input -------------------------------------------------------------------
INPUT_FILE = BASE_DIR / "dataloadimbalanceAC5" / "processed" / "adxl345_5sec_calibrated.csv"

# ---- Acquisition ---------------------------------------------------------
FS = 1559.0                     # Sampling frequency in Hz (measured, NOT assumed)
SENSOR_NAME = "ADXL345"
CONTROLLER_NAME = "ESP32"
UNITS = "g"

# ---- Time-domain waveform zoom window -----------------------------------
ZOOM_START = 0.0                # seconds
ZOOM_DURATION = 3.0             # seconds

# ---- Frequency-domain visualization zoom (NOT an analysis boundary) -----
LOW_FREQ_LIMIT = 100.0          # Hz - used only for zoomed FFT/PSD plots

# ---- Local spectral analysis ---------------------------------------------
LOCAL_BANDWIDTH_HZ = 100.0      # +/- window around each significant peak
MAX_SIGNIFICANT_PEAKS = 5       # top peaks selected for local analysis
MIN_PEAK_DISTANCE_HZ = 5.0      # minimum spacing enforced between peaks

# ---- Adaptive peak-detection parameters -----------------------------------
# The prominence threshold is derived from each dataset's OWN spectrum
# (robust noise-floor + spread), so it is never a fixed absolute number
# and adapts automatically to the amplitude scale of the current dataset.
PROMINENCE_SIGMA_MULT = 6.0     # threshold = noise_floor + MULT * robust_sigma
MIN_RELATIVE_PROMINENCE = 0.05  # additional floor: prominence >= 5% of spectrum max

# ---- Spectral roll-off -----------------------------------------------------
SPECTRAL_ROLLOFF_FRACTION = 0.85   # fraction of cumulative spectral energy

# ---- PSD (Welch) parameters ------------------------------------------------
PSD_WINDOW = "hann"
PSD_NPERSEG_TARGET = 4096        # will be clipped to available sample count
PSD_OVERLAP_FRACTION = 0.5

# ---- Spectrogram parameters ------------------------------------------------
SPEC_NPERSEG_TARGET = 512
SPEC_OVERLAP_FRACTION = 0.75

# ---- Autocorrelation --------------------------------------------------------
AUTOCORR_MAX_LAG_S = 1.0         # maximum lag shown, seconds (readability)

# ---- Spectral energy bands (Hz) --------------------------------------------
# Final band is automatically extended to the actual Nyquist frequency.
ENERGY_BAND_EDGES = [0, 10, 20, 50, 100, 200, 500]  # upper edge of last fixed band

# ---- Output directories -----------------------------------------------------
DATA_DIR = BASE_DIR / "dataloadimbalanceAC5"
PROCESSED_DIR = DATA_DIR / "processed"
RESULTS_DIR = DATA_DIR / "results"
PLOTS_DIR = RESULTS_DIR / "plots"
REPORTS_DIR = BASE_DIR / "dataloadimbalanceAC5" /  "reports"

# ---- Plot appearance ---------------------------------------------------------
PLOT_DPI = 170
FIGSIZE_SINGLE = (9.6, 3.85)     # full-width single plot (tuned so 3 fit cleanly per page)
FIGSIZE_WIDE = (9.6, 3.85)

AXIS_SIGNALS_ORIGINAL = ["X_g", "Y_g", "Z_g"]
AXIS_SIGNALS_AC = ["X_ac", "Y_ac", "Z_ac"]
ANALYSIS_SIGNALS = ["X_ac", "Y_ac", "Z_ac", "R_g"]   # used for features/FFT/PSD/etc.

plt.rcParams.update({
    "font.size": 10,
    "axes.titlesize": 11,
    "axes.labelsize": 10,
    "legend.fontsize": 9,
    "xtick.labelsize": 9,
    "ytick.labelsize": 9,
    "figure.autolayout": False,
})


def ensure_directories():
    for d in [DATA_DIR, PROCESSED_DIR, RESULTS_DIR, PLOTS_DIR, REPORTS_DIR]:
        d.mkdir(parents=True, exist_ok=True)


# ==============================================================================
# 1. DATA LOADING / VALIDATION / PREPROCESSING
# ==============================================================================

def load_data(input_file):
    """Load the processed accelerometer CSV. Raises clear errors on failure."""
    if not Path(input_file).exists():
        raise FileNotFoundError(f"Input file not found: {input_file}")

    try:
        df = pd.read_csv(input_file)
    except Exception as exc:
        raise ValueError(f"Could not read CSV '{input_file}': {exc}")

    if df.empty:
        raise ValueError(f"Input file '{input_file}' is empty.")

    return df


def validate_data(df, fs):
    """Validate required columns, numeric content, and sample sufficiency."""
    required_cols = ["X_g", "Y_g", "Z_g"]
    missing = [c for c in required_cols if c not in df.columns]
    if missing:
        raise ValueError(f"Missing required column(s): {missing}. "
                          f"Found columns: {list(df.columns)}")

    for col in required_cols:
        if not np.issubdtype(df[col].dtype, np.number):
            df[col] = pd.to_numeric(df[col], errors="coerce")

    n_nan = df[required_cols].isna().sum().sum()
    if n_nan > 0:
        print(f"WARNING: {n_nan} NaN value(s) found in {required_cols}. "
              f"Rows containing NaN will be dropped.")
        df = df.dropna(subset=required_cols).reset_index(drop=True)

    n_inf = np.isinf(df[required_cols].to_numpy()).sum()
    if n_inf > 0:
        print(f"WARNING: {n_inf} infinite value(s) found. Rows containing "
              f"infinite values will be dropped.")
        mask = np.isfinite(df[required_cols].to_numpy()).all(axis=1)
        df = df.loc[mask].reset_index(drop=True)

    n = len(df)
    if n < 16:
        raise ValueError(f"Insufficient samples for analysis (N={n}). "
                          f"Need at least 16 samples.")

    if fs is None or fs <= 0:
        raise ValueError(f"Invalid sampling frequency FS={fs}. Must be > 0.")

    # Minimum samples for a meaningful FFT (power-of-two not required, but
    # a very short record produces unusably coarse frequency resolution).
    if n < 64:
        print(f"WARNING: Only {n} samples available. Frequency resolution "
              f"will be coarse ({fs/n:.3f} Hz/bin).")

    return df.reset_index(drop=True)


def preprocess_data(df, fs):
    """
    Build the time vector and DC-removed AC vibration signals.
    Original processed signals (X_g, Y_g, Z_g) are preserved unmodified.
    """
    n = len(df)
    time_s = np.arange(n) / fs

    x_g = df["X_g"].to_numpy(dtype=float)
    y_g = df["Y_g"].to_numpy(dtype=float)
    z_g = df["Z_g"].to_numpy(dtype=float)

    x_ac = x_g - np.mean(x_g)
    y_ac = y_g - np.mean(y_g)
    z_ac = z_g - np.mean(z_g)

    out = pd.DataFrame({
        "Time_s": time_s,
        "X_g": x_g, "Y_g": y_g, "Z_g": z_g,
        "X_ac": x_ac, "Y_ac": y_ac, "Z_ac": z_ac,
    })
    return out


def calculate_resultant(df):
    """R_g = sqrt(X_ac^2 + Y_ac^2 + Z_ac^2)"""
    r_g = np.sqrt(df["X_ac"] ** 2 + df["Y_ac"] ** 2 + df["Z_ac"] ** 2)
    df = df.copy()
    df["R_g"] = r_g
    return df


def dataset_metadata(df, fs, input_file):
    n = len(df)
    duration = n / fs
    nyquist = fs / 2.0
    freq_res = fs / n
    return {
        "input_file": str(input_file),
        "dataset_name": Path(input_file).stem,
        "n_samples": n,
        "fs": fs,
        "duration_s": duration,
        "nyquist_hz": nyquist,
        "freq_resolution_hz": freq_res,
    }


# ==============================================================================
# 2. TIME-DOMAIN FEATURES
# ==============================================================================

def _safe_div(a, b):
    """Division that returns NaN instead of raising/inf on zero denominators."""
    with np.errstate(divide="ignore", invalid="ignore"):
        result = np.true_divide(a, b)
    if not np.isfinite(result):
        return np.nan
    return result


def calculate_time_features(x):
    """
    Calculate the full time-domain feature set for a single 1-D signal.
    Returns a dict of feature_name -> value. Undefined values (e.g. zero
    denominators) are returned as NaN rather than raising.
    """
    x = np.asarray(x, dtype=float)
    x = x[np.isfinite(x)]

    if x.size == 0:
        keys = ["Mean", "Median", "Std_Dev", "Variance", "RMS", "Maximum",
                "Minimum", "Absolute_Peak", "Peak_to_Peak", "Crest_Factor",
                "Shape_Factor", "Impulse_Factor", "Clearance_Factor",
                "Skewness", "Kurtosis", "Mean_Absolute_Value", "Energy",
                "Signal_Power", "Range", "MAD", "Q1", "Q3", "IQR",
                "Zero_Crossing_Rate"]
        return {k: np.nan for k in keys}

    mean_ = np.mean(x)
    median_ = np.median(x)
    std_ = np.std(x, ddof=0)
    var_ = np.var(x, ddof=0)
    rms_ = np.sqrt(np.mean(x ** 2))
    max_ = np.max(x)
    min_ = np.min(x)
    abs_peak = np.max(np.abs(x))
    p2p = max_ - min_
    mean_abs = np.mean(np.abs(x))
    sqrt_abs_mean = np.mean(np.sqrt(np.abs(x)))

    crest = _safe_div(abs_peak, rms_) if rms_ != 0 else np.nan
    shape = _safe_div(rms_, mean_abs) if mean_abs != 0 else np.nan
    impulse = _safe_div(abs_peak, mean_abs) if mean_abs != 0 else np.nan
    clearance_denom = sqrt_abs_mean ** 2
    clearance = _safe_div(abs_peak, clearance_denom) if clearance_denom != 0 else np.nan

    skewness = stats.skew(x, bias=False) if x.size >= 3 else np.nan
    kurtosis = stats.kurtosis(x, fisher=True, bias=False) if x.size >= 4 else np.nan

    energy = np.sum(x ** 2)
    power = np.mean(x ** 2)
    range_ = p2p

    mad = np.median(np.abs(x - median_))
    q1 = np.percentile(x, 25)
    q3 = np.percentile(x, 75)
    iqr = q3 - q1

    signs = np.sign(x)
    signs[signs == 0] = 1
    zero_crossings = np.sum(np.abs(np.diff(signs)) > 0)
    zcr = zero_crossings / (x.size - 1) if x.size > 1 else np.nan

    return {
        "Mean": mean_, "Median": median_, "Std_Dev": std_, "Variance": var_,
        "RMS": rms_, "Maximum": max_, "Minimum": min_,
        "Absolute_Peak": abs_peak, "Peak_to_Peak": p2p,
        "Crest_Factor": crest, "Shape_Factor": shape,
        "Impulse_Factor": impulse, "Clearance_Factor": clearance,
        "Skewness": skewness, "Kurtosis": kurtosis,
        "Mean_Absolute_Value": mean_abs, "Energy": energy,
        "Signal_Power": power, "Range": range_, "MAD": mad,
        "Q1": q1, "Q3": q3, "IQR": iqr, "Zero_Crossing_Rate": zcr,
    }


def build_time_feature_table(signals):
    """signals: dict of {name: array}. Returns DataFrame (features x signals)."""
    results = {name: calculate_time_features(sig) for name, sig in signals.items()}
    df = pd.DataFrame(results)
    df.index.name = "Feature"
    return df


# ==============================================================================
# 3. FFT / FREQUENCY-DOMAIN ANALYSIS
# ==============================================================================

def calculate_fft(x, fs):
    """
    Single-sided amplitude spectrum using a Hann window, covering the full
    0 Hz -> Nyquist range (never restricted to a sub-band here).
    Returns freqs, amplitude (window-corrected, physically meaningful units).
    """
    x = np.asarray(x, dtype=float)
    n = x.size
    window = np.hanning(n)
    windowed = x * window

    spectrum = rfft(windowed)
    freqs = rfftfreq(n, d=1.0 / fs)

    # Window amplitude correction (coherent gain), then single-sided doubling
    window_sum = np.sum(window)
    amp = np.abs(spectrum) * 2.0 / window_sum
    amp[0] = amp[0] / 2.0                      # DC bin: no doubling
    if n % 2 == 0 and len(amp) > 1:
        amp[-1] = amp[-1] / 2.0                # Nyquist bin: no doubling

    return freqs, amp


def calculate_power_spectrum(amp):
    """Power proportional to amplitude^2, used for spectral-moment features."""
    return amp ** 2


def estimate_noise_floor(power_no_dc):
    """
    Robust background/noise-floor estimate from the power spectrum
    (excluding DC), using the median (robust to sparse strong peaks) and
    a median-absolute-deviation-based robust spread.
    """
    if power_no_dc.size == 0:
        return 0.0, 0.0
    noise_floor = np.median(power_no_dc)
    mad = np.median(np.abs(power_no_dc - noise_floor))
    robust_sigma = 1.4826 * mad
    return noise_floor, robust_sigma


def calculate_frequency_features(freqs, amp, significant_peaks_count):
    """
    Frequency-domain feature set for one signal's full spectrum.
    Spectral moment features (centroid, bandwidth, variance, skew, kurtosis)
    are computed on the power spectrum (amp^2), excluding the DC bin.
    """
    power = calculate_power_spectrum(amp)

    # Exclude DC bin for moment-based / dominant-frequency calculations
    f_nz = freqs[1:]
    a_nz = amp[1:]
    p_nz = power[1:]

    total_power = np.sum(p_nz)

    if a_nz.size == 0 or total_power <= 0:
        return {
            "Dominant_Frequency_Hz": np.nan, "Dominant_Amplitude_g": np.nan,
            "Spectral_Centroid_Hz": np.nan, "Spectral_RMS_Frequency_Hz": np.nan,
            "Spectral_Bandwidth_Hz": np.nan, "Spectral_Energy": np.nan,
            "Spectral_Variance": np.nan, "Spectral_Skewness": np.nan,
            "Spectral_Kurtosis": np.nan, "Spectral_Flatness": np.nan,
            "Spectral_Entropy": np.nan, "Spectral_Rolloff_Hz": np.nan,
            "Spectral_Peak_Power": np.nan,
            "Number_of_Significant_Peaks": significant_peaks_count,
        }

    dom_idx = np.argmax(a_nz)
    dominant_freq = f_nz[dom_idx]
    dominant_amp = a_nz[dom_idx]

    centroid = np.sum(f_nz * p_nz) / total_power
    rms_freq = np.sqrt(np.sum((f_nz ** 2) * p_nz) / total_power)
    variance = np.sum(((f_nz - centroid) ** 2) * p_nz) / total_power
    bandwidth = np.sqrt(variance)

    if bandwidth > 0:
        skewness = (np.sum(((f_nz - centroid) ** 3) * p_nz) / total_power) / (bandwidth ** 3)
        kurtosis = (np.sum(((f_nz - centroid) ** 4) * p_nz) / total_power) / (bandwidth ** 4)
    else:
        skewness = np.nan
        kurtosis = np.nan

    spectral_energy = total_power

    p_positive = p_nz[p_nz > 0]
    if p_positive.size > 0:
        geo_mean = np.exp(np.mean(np.log(p_positive)))
        arith_mean = np.mean(p_positive)
        flatness = _safe_div(geo_mean, arith_mean) if arith_mean != 0 else np.nan
    else:
        flatness = np.nan

    p_dist = p_nz / total_power
    p_dist_nz = p_dist[p_dist > 0]
    entropy_raw = -np.sum(p_dist_nz * np.log2(p_dist_nz)) if p_dist_nz.size > 0 else np.nan
    max_entropy = np.log2(p_dist.size) if p_dist.size > 1 else np.nan
    entropy_norm = _safe_div(entropy_raw, max_entropy) if max_entropy else np.nan

    cumulative_power = np.cumsum(p_nz)
    rolloff_idx = np.searchsorted(cumulative_power, SPECTRAL_ROLLOFF_FRACTION * total_power)
    rolloff_idx = min(rolloff_idx, f_nz.size - 1)
    rolloff_freq = f_nz[rolloff_idx]

    spectral_peak_power = np.max(p_nz)

    return {
        "Dominant_Frequency_Hz": dominant_freq,
        "Dominant_Amplitude_g": dominant_amp,
        "Spectral_Centroid_Hz": centroid,
        "Spectral_RMS_Frequency_Hz": rms_freq,
        "Spectral_Bandwidth_Hz": bandwidth,
        "Spectral_Energy": spectral_energy,
        "Spectral_Variance": variance,
        "Spectral_Skewness": skewness,
        "Spectral_Kurtosis": kurtosis,
        "Spectral_Flatness": flatness,
        "Spectral_Entropy": entropy_norm,
        "Spectral_Rolloff_Hz": rolloff_freq,
        "Spectral_Peak_Power": spectral_peak_power,
        "Number_of_Significant_Peaks": significant_peaks_count,
    }


# ==============================================================================
# 4. ADAPTIVE FULL-SPECTRUM SIGNIFICANT PEAK DETECTION
# ==============================================================================

def detect_significant_peaks(freqs, amp, fs, n_samples,
                              min_peak_distance_hz=MIN_PEAK_DISTANCE_HZ,
                              max_peaks=MAX_SIGNIFICANT_PEAKS):
    """
    Adaptive significant-peak detection over the COMPLETE 0 Hz -> Nyquist
    spectrum (DC excluded). The prominence threshold is derived from this
    dataset's own robust noise floor, so it automatically adapts to the
    amplitude scale of whatever motor condition produced this spectrum
    (never a fixed absolute number such as 0.001).

    Procedure:
      1. Work on the full single-sided amplitude spectrum, DC excluded.
      2. Estimate a robust background/noise level from the power spectrum
         (median + MAD-based robust sigma).
      3. Adaptive minimum prominence = noise_floor + K * robust_sigma,
         floored by a small fraction of the spectrum's own peak power so
         extremely quiet/flat spectra still get a sane threshold.
      4. Run scipy.signal.find_peaks with that prominence and a minimum
         bin separation derived from MIN_PEAK_DISTANCE_HZ / freq_resolution.
      5. Rank surviving peaks by prominence (descending) and keep the
         strongest max_peaks.

    Returns a list of dicts, one per accepted peak, each containing index,
    frequency, amplitude, and prominence (in power units and amplitude
    units), sorted by rank (strongest first).
    """
    freq_res = fs / n_samples

    freqs_nz = freqs[1:]
    amp_nz = amp[1:]
    power_nz = amp_nz ** 2

    if amp_nz.size < 3:
        return []

    noise_floor_p, robust_sigma_p = estimate_noise_floor(power_nz)
    adaptive_min_prom_power = noise_floor_p + PROMINENCE_SIGMA_MULT * robust_sigma_p

    min_distance_bins = max(1, int(round(min_peak_distance_hz / freq_res)))

    # The relative-amplitude floor must be scaled against genuine LOCAL-MAXIMUM
    # candidates, not the raw spectrum maximum. A signal such as R_g (a
    # non-negative resultant magnitude) is not zero-mean, so spectral leakage
    # from windowing can dump a very large amount of energy into the bin(s)
    # immediately next to DC. That leakage bin sits at the very edge of the
    # DC-excluded array and can never itself be selected as a peak (find_peaks
    # requires two-sided neighbors), but if it were allowed to set the
    # "relative to spectrum max" floor it would silently suppress every real
    # harmonic elsewhere. So the relative floor is scaled against the
    # strongest actual local-maximum candidate instead.
    candidate_idx, _ = find_peaks(power_nz, distance=min_distance_bins)
    if candidate_idx.size == 0:
        return []
    candidate_max_power = np.max(power_nz[candidate_idx])
    relative_floor = MIN_RELATIVE_PROMINENCE * candidate_max_power

    min_prominence_power = max(adaptive_min_prom_power, relative_floor)

    peak_idx, props = find_peaks(
        power_nz,
        prominence=min_prominence_power,
        distance=min_distance_bins,
    )

    if peak_idx.size == 0:
        return []

    prominences = props["prominences"]
    order = np.argsort(prominences)[::-1]
    peak_idx = peak_idx[order]
    prominences = prominences[order]

    peak_idx = peak_idx[:max_peaks]
    prominences = prominences[:max_peaks]

    results = []
    for rank, (idx, prom_power) in enumerate(zip(peak_idx, prominences), start=1):
        freq_hz = freqs_nz[idx]
        amp_g = amp_nz[idx]
        # Convert power-domain prominence back to an approximate amplitude-
        # domain prominence for reporting (sqrt of power prominence, signed
        # consistently with the amplitude at this bin).
        prom_amp = math.sqrt(prom_power) if prom_power > 0 else 0.0
        results.append({
            "rank": rank,
            "index_in_nz": int(idx),
            "frequency_hz": freq_hz,
            "amplitude_g": amp_g,
            "prominence_power": prom_power,
            "prominence_amplitude": prom_amp,
            "noise_floor_power": noise_floor_p,
            "min_prominence_power_used": min_prominence_power,
        })

    return results


# ==============================================================================
# 5. LOCAL (+/-100 Hz) SPECTRAL ANALYSIS AROUND SIGNIFICANT PEAKS
# ==============================================================================

def calculate_local_spectral_features(freqs, amp, peak, nyquist,
                                       all_peaks, bandwidth_hz=LOCAL_BANDWIDTH_HZ):
    """
    For a single detected significant peak, compute local spectral features
    within an automatically clipped +/- bandwidth_hz window around it.
    This is an additional zoomed analysis; it never replaces the full
    spectrum.
    """
    fp = peak["frequency_hz"]
    lower = max(0.0, fp - bandwidth_hz)
    upper = min(nyquist, fp + bandwidth_hz)

    mask = (freqs >= lower) & (freqs <= upper)
    f_local = freqs[mask]
    a_local = amp[mask]
    p_local = a_local ** 2

    total_p = np.sum(p_local)
    if total_p > 0 and f_local.size > 0:
        local_centroid = np.sum(f_local * p_local) / total_p
        local_rms_freq = np.sqrt(np.sum((f_local ** 2) * p_local) / total_p)
        local_bandwidth = np.sqrt(np.sum(((f_local - local_centroid) ** 2) * p_local) / total_p)
        local_energy = total_p
    else:
        local_centroid = np.nan
        local_rms_freq = np.nan
        local_bandwidth = np.nan
        local_energy = np.nan

    # Nearby significant peaks (other detected peaks that fall in this window)
    nearby = [p for p in all_peaks
              if p is not peak and lower <= p["frequency_hz"] <= upper]
    nearby_freqs = sorted(p["frequency_hz"] for p in nearby)
    if nearby_freqs:
        spacing = min(abs(f - fp) for f in nearby_freqs)
    else:
        spacing = np.nan

    return {
        "Local_Lower_Hz": lower,
        "Local_Upper_Hz": upper,
        "Local_Energy": local_energy,
        "Local_Centroid_Hz": local_centroid,
        "Local_Bandwidth_Hz": local_bandwidth,
        "Local_RMS_Frequency_Hz": local_rms_freq,
        "Nearby_Significant_Peaks": len(nearby),
        "Nearest_Peak_Spacing_Hz": spacing,
        "f_local": f_local,
        "a_local": a_local,
    }


# ==============================================================================
# 6. POWER SPECTRAL DENSITY (Welch)
# ==============================================================================

def calculate_psd(x, fs):
    """Welch PSD over the full valid range 0 -> Nyquist."""
    n = len(x)
    nperseg = int(min(PSD_NPERSEG_TARGET, n))
    nperseg = max(nperseg, 8)
    noverlap = int(nperseg * PSD_OVERLAP_FRACTION)

    f, pxx = welch(x, fs=fs, window=PSD_WINDOW, nperseg=nperseg,
                   noverlap=noverlap, scaling="density")
    params = {"window": PSD_WINDOW, "nperseg": nperseg, "noverlap": noverlap}
    return f, pxx, params


# ==============================================================================
# 7. SPECTROGRAM
# ==============================================================================

def calculate_spectrogram(x, fs):
    n = len(x)
    nperseg = int(min(SPEC_NPERSEG_TARGET, n))
    nperseg = max(nperseg, 8)
    noverlap = int(nperseg * SPEC_OVERLAP_FRACTION)

    f, t, sxx = sp_spectrogram(x, fs=fs, window="hann", nperseg=nperseg,
                                noverlap=noverlap, scaling="density")
    params = {"nperseg": nperseg, "noverlap": noverlap}
    return f, t, sxx, params


# ==============================================================================
# 8. AUTOCORRELATION
# ==============================================================================

def calculate_autocorrelation(x, fs, max_lag_s=AUTOCORR_MAX_LAG_S):
    x = np.asarray(x, dtype=float)
    x = x - np.mean(x)
    n = x.size

    full_corr = np.correlate(x, x, mode="full")
    mid = full_corr.size // 2
    corr = full_corr[mid:]
    if corr[0] != 0:
        corr = corr / corr[0]

    max_lag_samples = min(n - 1, int(round(max_lag_s * fs)))
    lags_s = np.arange(max_lag_samples + 1) / fs
    return lags_s, corr[:max_lag_samples + 1]


# ==============================================================================
# 9. EMPIRICAL CDF
# ==============================================================================

def calculate_ecdf(x):
    x = np.asarray(x, dtype=float)
    x_sorted = np.sort(x)
    n = x_sorted.size
    prob = np.arange(1, n + 1) / n
    return x_sorted, prob


# ==============================================================================
# 10. SPECTRAL ENERGY (overall + frequency bands)
# ==============================================================================

def calculate_spectral_energy(freqs, amp, nyquist, band_edges=ENERGY_BAND_EDGES):
    """
    Total spectral energy (sum of power, DC excluded) plus energy within
    fixed visualization bands. The final band is extended to the actual
    Nyquist frequency. These bands are for visualization only and are not
    used as an analysis boundary elsewhere.
    """
    power = amp ** 2
    freqs_nz = freqs[1:]
    power_nz = power[1:]

    total_energy = np.sum(power_nz)

    edges = list(band_edges)
    if edges[-1] < nyquist:
        edges = edges + [nyquist]
    else:
        edges = [e for e in edges if e < nyquist] + [nyquist]

    bands = []
    for lo, hi in zip(edges[:-1], edges[1:]):
        mask = (freqs_nz >= lo) & (freqs_nz < hi)
        band_energy = np.sum(power_nz[mask])
        bands.append({
            "Band_Low_Hz": lo,
            "Band_High_Hz": hi,
            "Energy": band_energy,
            "Fraction_of_Total": _safe_div(band_energy, total_energy) if total_energy else np.nan,
        })

    return total_energy, pd.DataFrame(bands)


# ==============================================================================
# 11. PLOTTING HELPERS
# ==============================================================================

def _save_fig(fig, path):
    fig.savefig(path, dpi=PLOT_DPI, bbox_inches="tight")
    plt.close(fig)


def _style_axes(ax):
    ax.grid(True, alpha=0.3, linewidth=0.6)
    ax.spines["top"].set_visible(False)
    ax.spines["right"].set_visible(False)


def generate_time_plots(df, fs, zoom_start, zoom_duration, plots_dir):
    """Full + zoomed waveform plots for original processed signals X_g,Y_g,Z_g."""
    paths = {}
    t = df["Time_s"].to_numpy()
    zoom_end = zoom_start + zoom_duration
    zoom_mask = (t >= zoom_start) & (t <= zoom_end)

    for col in AXIS_SIGNALS_ORIGINAL:
        y = df[col].to_numpy()

        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(t, y, linewidth=0.5, color="#1f4e8c")
        ax.set_title(f"{col} \u2014 Full Time-Domain Waveform")
        ax.set_xlabel("Time (s)")
        ax.set_ylabel(f"Acceleration, {col} (g)")
        ax.set_xlim(t[0], t[-1])
        _style_axes(ax)
        p_full = plots_dir / f"time_{col}_full.png"
        _save_fig(fig, p_full)

        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(t[zoom_mask], y[zoom_mask], linewidth=0.9, color="#1f4e8c")
        ax.set_title(f"{col} \u2014 Time-Domain Waveform ({zoom_start:.0f}\u2013{zoom_end:.0f} s)")
        ax.set_xlabel("Time (s)")
        ax.set_ylabel(f"Acceleration, {col} (g)")
        if np.any(zoom_mask):
            ax.set_xlim(zoom_start, min(zoom_end, t[-1]))
        _style_axes(ax)
        p_zoom = plots_dir / f"time_{col}_zoom.png"
        _save_fig(fig, p_zoom)

        paths[col] = {"full": p_full, "zoom": p_zoom}

    return paths


def generate_ac_plots(df, fs, zoom_start, zoom_duration, plots_dir):
    """Full + zoomed waveform plots for AC signals X_ac,Y_ac,Z_ac and R_g."""
    paths = {}
    t = df["Time_s"].to_numpy()
    zoom_end = zoom_start + zoom_duration
    zoom_mask = (t >= zoom_start) & (t <= zoom_end)

    for col in AXIS_SIGNALS_AC + ["R_g"]:
        y = df[col].to_numpy()
        color = "#b5451b" if col != "R_g" else "#2f7d32"

        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(t, y, linewidth=0.5, color=color)
        label = f"{col} \u2014 Full AC Vibration Waveform" if col != "R_g" \
            else f"{col} \u2014 Full Resultant Vibration Waveform"
        ax.set_title(label)
        ax.set_xlabel("Time (s)")
        ax.set_ylabel(f"Acceleration, {col} (g)")
        ax.set_xlim(t[0], t[-1])
        _style_axes(ax)
        p_full = plots_dir / f"ac_{col}_full.png"
        _save_fig(fig, p_full)

        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(t[zoom_mask], y[zoom_mask], linewidth=0.9, color=color)
        label_z = f"{col} \u2014 AC Vibration Waveform ({zoom_start:.0f}\u2013{zoom_end:.0f} s)" if col != "R_g" \
            else f"{col} \u2014 Resultant Vibration Waveform ({zoom_start:.0f}\u2013{zoom_end:.0f} s)"
        ax.set_title(label_z)
        ax.set_xlabel("Time (s)")
        ax.set_ylabel(f"Acceleration, {col} (g)")
        if np.any(zoom_mask):
            ax.set_xlim(zoom_start, min(zoom_end, t[-1]))
        _style_axes(ax)
        p_zoom = plots_dir / f"ac_{col}_zoom.png"
        _save_fig(fig, p_zoom)

        paths[col] = {"full": p_full, "zoom": p_zoom}

    return paths


def generate_fft_plots(fft_data, nyquist, low_freq_limit, plots_dir):
    """
    fft_data: {signal_name: (freqs, amp)}
    Generates full-spectrum (0->Nyquist) and low-frequency-zoom plots.
    The zoom is a VISUALIZATION only; full spectrum remains primary.
    """
    paths = {}
    for name, (freqs, amp) in fft_data.items():
        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(freqs, amp, linewidth=0.6, color="#1f4e8c")
        ax.set_title(f"{name} \u2014 Full Spectrum FFT (0\u2013{nyquist:.1f} Hz)")
        ax.set_xlabel("Frequency (Hz)")
        ax.set_ylabel(f"Amplitude, {name} (g)")
        ax.set_xlim(0, nyquist)
        _style_axes(ax)
        p_full = plots_dir / f"fft_{name}_full.png"
        _save_fig(fig, p_full)

        zoom_mask = freqs <= low_freq_limit
        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.plot(freqs[zoom_mask], amp[zoom_mask], linewidth=0.8, color="#1f4e8c")
        ax.set_title(f"{name} \u2014 FFT Zoom (0\u2013{low_freq_limit:.0f} Hz)")
        ax.set_xlabel("Frequency (Hz)")
        ax.set_ylabel(f"Amplitude, {name} (g)")
        ax.set_xlim(0, low_freq_limit)
        _style_axes(ax)
        p_zoom = plots_dir / f"fft_{name}_zoom.png"
        _save_fig(fig, p_zoom)

        paths[name] = {"full": p_full, "zoom": p_zoom}

    return paths


def generate_local_spectral_plots(name, freqs, amp, peaks, local_features_list, plots_dir):
    """One local +/-100 Hz plot per significant peak for a given signal."""
    out = []
    for peak, local_feat in zip(peaks, local_features_list):
        fp = peak["frequency_hz"]
        lower = local_feat["Local_Lower_Hz"]
        upper = local_feat["Local_Upper_Hz"]
        f_local = local_feat["f_local"]
        a_local = local_feat["a_local"]

        fig, ax = plt.subplots(figsize=(9.4, 3.7))
        ax.plot(f_local, a_local, linewidth=0.8, color="#1f4e8c")
        ax.axvline(fp, color="#c0392b", linestyle="--", linewidth=1.0)
        ax.plot([fp], [peak["amplitude_g"]], marker="o", color="#c0392b", markersize=5, zorder=5)

        annotation = (f"f = {fp:.2f} Hz\n"
                      f"amp = {peak['amplitude_g']:.5g} g\n"
                      f"prom = {peak['prominence_amplitude']:.5g} g")
        ax.annotate(annotation, xy=(fp, peak["amplitude_g"]),
                    xytext=(0.98, 0.95), textcoords="axes fraction",
                    ha="right", va="top", fontsize=8,
                    bbox=dict(boxstyle="round,pad=0.35", fc="white", ec="#999999", alpha=0.9))

        ax.set_title(f"{name} \u2014 Local Spectrum Around {fp:.1f} Hz "
                     f"(Rank {peak['rank']})")
        ax.set_xlabel("Frequency (Hz)")
        ax.set_ylabel(f"Amplitude, {name} (g)")
        ax.set_xlim(lower, upper)
        _style_axes(ax)

        p = plots_dir / f"local_{name}_rank{peak['rank']}_{fp:.1f}Hz.png"
        _save_fig(fig, p)
        out.append({"path": p, "peak": peak, "local_feat": local_feat})

    return out


def generate_psd_plots(psd_data, nyquist, low_freq_limit, plots_dir):
    """psd_data: {name: (f, pxx)}"""
    paths = {}
    for name, (f, pxx) in psd_data.items():
        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.semilogy(f, pxx + 1e-30, linewidth=0.7, color="#1f4e8c")
        ax.set_title(f"{name} \u2014 Power Spectral Density (0\u2013{nyquist:.1f} Hz)")
        ax.set_xlabel("Frequency (Hz)")
        ax.set_ylabel("PSD (g\u00b2/Hz)")
        ax.set_xlim(0, nyquist)
        _style_axes(ax)
        p_full = plots_dir / f"psd_{name}_full.png"
        _save_fig(fig, p_full)

        mask = f <= low_freq_limit
        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        ax.semilogy(f[mask], pxx[mask] + 1e-30, linewidth=0.9, color="#1f4e8c")
        ax.set_title(f"{name} \u2014 PSD Zoom (0\u2013{low_freq_limit:.0f} Hz)")
        ax.set_xlabel("Frequency (Hz)")
        ax.set_ylabel("PSD (g\u00b2/Hz)")
        ax.set_xlim(0, low_freq_limit)
        _style_axes(ax)
        p_zoom = plots_dir / f"psd_{name}_zoom.png"
        _save_fig(fig, p_zoom)

        paths[name] = {"full": p_full, "zoom": p_zoom}

    return paths


def generate_spectrogram_plots(spec_data, plots_dir):
    """spec_data: {name: (f, t, sxx)}"""
    paths = {}
    for name, (f, t, sxx) in spec_data.items():
        sxx_db = 10 * np.log10(sxx + 1e-30)
        fig, ax = plt.subplots(figsize=FIGSIZE_SINGLE)
        mesh = ax.pcolormesh(t, f, sxx_db, shading="gouraud", cmap="viridis")
        ax.set_title(f"{name} \u2014 Spectrogram")
        ax.set_xlabel("Time (s)")
        ax.set_ylabel("Frequency (Hz)")
        cbar = fig.colorbar(mesh, ax=ax)
        cbar.set_label("Power/Frequency (dB/Hz)")
        p = plots_dir / f"spectrogram_{name}.png"
        _save_fig(fig, p)
        paths[name] = p

    return paths


def generate_autocorrelation_plots(autocorr_data, plots_dir):
    """autocorr_data: {name: (lags_s, corr)}"""
    paths = {}
    for name, (lags, corr) in autocorr_data.items():
        fig, ax = plt.subplots(figsize=FIGSIZE_WIDE)
        ax.plot(lags, corr, linewidth=0.8, color="#1f4e8c")
        ax.axhline(0, color="#999999", linewidth=0.6)
        ax.set_title(f"{name} \u2014 Autocorrelation")
        ax.set_xlabel("Lag (s)")
        ax.set_ylabel("Normalized Autocorrelation")
        ax.set_xlim(lags[0], lags[-1])
        _style_axes(ax)
        p = plots_dir / f"autocorr_{name}.png"
        _save_fig(fig, p)
        paths[name] = p

    return paths


def generate_ecdf_plots(ecdf_data, plots_dir):
    """ecdf_data: {name: (x_sorted, prob)}"""
    paths = {}
    for name, (x_sorted, prob) in ecdf_data.items():
        fig, ax = plt.subplots(figsize=FIGSIZE_WIDE)
        ax.plot(x_sorted, prob, linewidth=1.0, color="#1f4e8c")
        ax.set_title(f"{name} \u2014 Empirical CDF")
        ax.set_xlabel(f"Acceleration, {name} (g)")
        ax.set_ylabel("Cumulative Probability")
        ax.set_ylim(0, 1)
        _style_axes(ax)
        p = plots_dir / f"ecdf_{name}.png"
        _save_fig(fig, p)
        paths[name] = p

    return paths


def generate_spectral_energy_plot(energy_band_tables, plots_dir):
    """
    energy_band_tables: {name: DataFrame with Band_Low_Hz/Band_High_Hz/Energy}
    One combined grouped-bar chart (bands on x-axis, one bar group per
    signal) to avoid generating four near-identical redundant figures.
    """
    names = list(energy_band_tables.keys())
    ref_df = energy_band_tables[names[0]]
    band_labels = [f"{int(r.Band_Low_Hz)}\u2013{r.Band_High_Hz:.0f}"
                   for r in ref_df.itertuples()]
    n_bands = len(band_labels)
    n_signals = len(names)

    x = np.arange(n_bands)
    width = 0.8 / n_signals
    colors_list = ["#1f4e8c", "#b5451b", "#2f7d32", "#8e44ad"]

    fig, ax = plt.subplots(figsize=(9.6, 4.8))
    for i, name in enumerate(names):
        energies = energy_band_tables[name]["Energy"].to_numpy()
        ax.bar(x + i * width - 0.4 + width / 2, energies + 1e-30, width,
               label=name, color=colors_list[i % len(colors_list)])

    ax.set_yscale("log")
    ax.set_xticks(x)
    ax.set_xticklabels(band_labels, rotation=0)
    ax.set_xlabel("Frequency Band (Hz)")
    ax.set_ylabel("Band Spectral Energy (g\u00b2, log scale)")
    ax.set_title("Frequency-Band Spectral Energy Distribution")
    ax.legend(ncol=4, loc="upper center", bbox_to_anchor=(0.5, -0.15), frameon=False)
    _style_axes(ax)

    p = plots_dir / "spectral_energy_bands.png"
    _save_fig(fig, p)
    return p


# ==============================================================================
# 12. CSV OUTPUTS
# ==============================================================================

def generate_csv_outputs(df_processed, time_feat_df, freq_feat_df,
                          peaks_by_signal, local_rows, results_dir):
    processed_path = results_dir / "processed_analysis.csv"
    df_processed.to_csv(processed_path, index=False)

    time_feat_path = results_dir / "time_domain_features.csv"
    time_feat_df.to_csv(time_feat_path)

    freq_feat_path = results_dir / "frequency_domain_features.csv"
    freq_feat_df.to_csv(freq_feat_path)

    peak_rows = []
    for signal, peaks in peaks_by_signal.items():
        for p in peaks:
            peak_rows.append({
                "Signal": signal,
                "Rank": p["rank"],
                "Frequency_Hz": p["frequency_hz"],
                "Amplitude_g": p["amplitude_g"],
                "Prominence_Power": p["prominence_power"],
                "Prominence_Amplitude_g": p["prominence_amplitude"],
                "Equivalent_RPM_if_1x": p["frequency_hz"] * 60.0,
                "Noise_Floor_Power": p["noise_floor_power"],
                "Min_Prominence_Power_Used": p["min_prominence_power_used"],
            })
    spectral_peaks_df = pd.DataFrame(peak_rows)
    spectral_peaks_path = results_dir / "spectral_peaks.csv"
    spectral_peaks_df.to_csv(spectral_peaks_path, index=False)

    local_df = pd.DataFrame(local_rows)
    local_path = results_dir / "local_spectral_analysis.csv"
    if not local_df.empty:
        local_df = local_df[[
            "Signal", "Peak_Rank", "Peak_Frequency_Hz", "Peak_Amplitude_g",
            "Peak_Prominence", "Local_Lower_Hz", "Local_Upper_Hz",
            "Local_Energy", "Local_Centroid_Hz", "Local_Bandwidth_Hz",
            "Local_RMS_Frequency_Hz",
        ]]
    local_df.to_csv(local_path, index=False)

    return {
        "processed_analysis": processed_path,
        "time_domain_features": time_feat_path,
        "frequency_domain_features": freq_feat_path,
        "spectral_peaks": spectral_peaks_path,
        "local_spectral_analysis": local_path,
    }


# ==============================================================================
# 13. PDF REPORT GENERATION
# ==============================================================================

PAGE_W, PAGE_H = A4
MARGIN = 18 * mm
CONTENT_W = PAGE_W - 2 * MARGIN


def _fmt(v, sig=5):
    """Compact, readable numeric formatting for report tables."""
    if v is None:
        return "N/A"
    try:
        if isinstance(v, str):
            return v
        if pd.isna(v):
            return "N/A"
    except (TypeError, ValueError):
        pass
    if isinstance(v, (int, np.integer)):
        return str(int(v))
    av = abs(v)
    if av != 0 and (av < 1e-3 or av >= 1e5):
        return f"{v:.{sig-1}e}"
    return f"{v:.{sig}g}"


def _build_styles():
    styles = getSampleStyleSheet()
    styles.add(ParagraphStyle(
        name="ReportTitle", parent=styles["Title"], fontSize=20, leading=24,
        spaceAfter=6, alignment=TA_CENTER, textColor=colors.HexColor("#12233f")))
    styles.add(ParagraphStyle(
        name="ReportSubtitle", parent=styles["Normal"], fontSize=11.5,
        leading=15, alignment=TA_CENTER, textColor=colors.HexColor("#3a3a3a")))
    styles.add(ParagraphStyle(
        name="SectionHeading", parent=styles["Heading1"], fontSize=14.5,
        leading=18, spaceBefore=6, spaceAfter=8,
        textColor=colors.HexColor("#12233f"),
        borderColor=colors.HexColor("#12233f"), borderWidth=0,
    ))
    styles.add(ParagraphStyle(
        name="SubHeading", parent=styles["Heading2"], fontSize=11.5,
        leading=14, spaceBefore=10, spaceAfter=5,
        textColor=colors.HexColor("#1f4e8c")))
    styles.add(ParagraphStyle(
        name="BodyTextJ", parent=styles["Normal"], fontSize=9.6, leading=13.5,
        alignment=TA_LEFT, spaceAfter=6))
    styles.add(ParagraphStyle(
        name="Caption", parent=styles["Normal"], fontSize=8.6, leading=11,
        alignment=TA_CENTER, textColor=colors.HexColor("#444444"),
        spaceBefore=2, spaceAfter=10))
    styles.add(ParagraphStyle(
        name="CoverMeta", parent=styles["Normal"], fontSize=11, leading=17,
        alignment=TA_CENTER))
    return styles


def _header_footer(canvas_obj, doc, dataset_name):
    canvas_obj.saveState()
    canvas_obj.setStrokeColor(colors.HexColor("#c9c9c9"))
    canvas_obj.setLineWidth(0.5)

    # Header
    canvas_obj.setFont("Helvetica", 8)
    canvas_obj.setFillColor(colors.HexColor("#666666"))
    canvas_obj.drawString(MARGIN, PAGE_H - 12 * mm,
                           "Motor Vibration Experimental Data Analysis")
    canvas_obj.drawRightString(PAGE_W - MARGIN, PAGE_H - 12 * mm, dataset_name)
    canvas_obj.line(MARGIN, PAGE_H - 13 * mm, PAGE_W - MARGIN, PAGE_H - 13 * mm)

    # Footer
    canvas_obj.line(MARGIN, 14 * mm, PAGE_W - MARGIN, 14 * mm)
    canvas_obj.setFont("Helvetica", 8)
    canvas_obj.drawString(MARGIN, 10 * mm, "Data-centric experimental record \u2014 no automatic interpretation")
    canvas_obj.drawRightString(PAGE_W - MARGIN, 10 * mm, f"Page {doc.page}")
    canvas_obj.restoreState()


def _img_flowable(path, max_width_mm=170, max_height_mm=110):
    """Scale an image to fit within given bounds, preserving aspect ratio."""
    from PIL import Image as PILImage
    with PILImage.open(path) as im:
        w_px, h_px = im.size
    aspect = h_px / w_px
    width = max_width_mm * mm
    height = width * aspect
    max_h = max_height_mm * mm
    if height > max_h:
        height = max_h
        width = height / aspect
    return Image(str(path), width=width, height=height)


def _add_image_block(story, styles, path, caption, max_width_mm=172, max_height_mm=118):
    img = _img_flowable(path, max_width_mm, max_height_mm)
    cap = Paragraph(caption, styles["Caption"])
    story.append(KeepTogether([img, cap]))


def _df_to_table(df, styles, col_widths=None, index_label="Feature", font_size=8.0):
    """Convert a DataFrame (features as rows) into a styled ReportLab Table."""
    header = [index_label] + list(df.columns)
    data = [header]
    for idx, row in df.iterrows():
        label = str(idx).replace("_", " ")
        data.append([label] + [_fmt(v) for v in row.values])

    n_cols = len(header)
    if col_widths is None:
        first_w = CONTENT_W * 0.30
        other_w = (CONTENT_W - first_w) / (n_cols - 1)
        col_widths = [first_w] + [other_w] * (n_cols - 1)

    table = Table(data, colWidths=col_widths, repeatRows=1)
    table.setStyle(TableStyle([
        ("BACKGROUND", (0, 0), (-1, 0), colors.HexColor("#12233f")),
        ("TEXTCOLOR", (0, 0), (-1, 0), colors.white),
        ("FONTNAME", (0, 0), (-1, 0), "Helvetica-Bold"),
        ("FONTSIZE", (0, 0), (-1, -1), font_size),
        ("FONTNAME", (0, 1), (0, -1), "Helvetica-Bold"),
        ("ROWBACKGROUNDS", (0, 1), (-1, -1), [colors.white, colors.HexColor("#f2f5fa")]),
        ("GRID", (0, 0), (-1, -1), 0.4, colors.HexColor("#c9c9c9")),
        ("ALIGN", (1, 0), (-1, -1), "CENTER"),
        ("ALIGN", (0, 0), (0, -1), "LEFT"),
        ("VALIGN", (0, 0), (-1, -1), "MIDDLE"),
        ("TOPPADDING", (0, 0), (-1, -1), 3),
        ("BOTTOMPADDING", (0, 0), (-1, -1), 3),
    ]))
    return table


def generate_pdf_report(meta, df_processed, time_feat_df, freq_feat_df,
                         time_plots, ac_plots, fft_plots, psd_plots,
                         spectrogram_plots, autocorr_plots, ecdf_plots,
                         energy_plot_path, energy_band_tables,
                         peaks_by_signal, local_plot_results,
                         psd_params, spec_params, autocorr_max_lag_s,
                         reports_dir):
    styles = _build_styles()
    dataset_name = meta["dataset_name"]
    out_path = reports_dir / f"{dataset_name}_report.pdf"

    doc = SimpleDocTemplate(
        str(out_path), pagesize=A4,
        leftMargin=MARGIN, rightMargin=MARGIN,
        topMargin=MARGIN + 4 * mm, bottomMargin=MARGIN,
        title=f"Motor Vibration Analysis - {dataset_name}",
        author="Experimental Vibration Analysis System",
    )

    def _on_page(canvas_obj, doc_obj):
        _header_footer(canvas_obj, doc_obj, dataset_name)

    story = []

    # -------------------- COVER PAGE --------------------
    story.append(Spacer(1, 42 * mm))
    story.append(Paragraph("MOTOR VIBRATION EXPERIMENTAL<br/>DATA ANALYSIS", styles["ReportTitle"]))
    story.append(Spacer(1, 8 * mm))
    story.append(HRFlowable(width="60%", thickness=1, color=colors.HexColor("#c9c9c9"),
                             hAlign="CENTER", spaceAfter=10 * mm))

    cover_rows = [
        ["Dataset", dataset_name],
        ["Sensor", SENSOR_NAME],
        ["Controller", CONTROLLER_NAME],
        ["Sampling Frequency", f"{meta['fs']:.2f} Hz"],
        ["Number of Samples", f"{meta['n_samples']}"],
        ["Duration", f"{meta['duration_s']:.3f} s"],
    ]
    cover_table = Table(cover_rows, colWidths=[55 * mm, 75 * mm], hAlign="CENTER")
    cover_table.setStyle(TableStyle([
        ("FONTSIZE", (0, 0), (-1, -1), 11),
        ("FONTNAME", (0, 0), (0, -1), "Helvetica-Bold"),
        ("TEXTCOLOR", (0, 0), (0, -1), colors.HexColor("#12233f")),
        ("TOPPADDING", (0, 0), (-1, -1), 6),
        ("BOTTOMPADDING", (0, 0), (-1, -1), 6),
        ("LINEBELOW", (0, 0), (-1, -2), 0.4, colors.HexColor("#dddddd")),
    ]))
    story.append(cover_table)
    story.append(PageBreak())

    # -------------------- 1. DATASET INFORMATION --------------------
    story.append(Paragraph("1. Dataset Information", styles["SectionHeading"]))
    info_rows = [
        ["Parameter", "Value"],
        ["Input file", Path(meta["input_file"]).name],
        ["Samples", f"{meta['n_samples']}"],
        ["Sampling frequency", f"{meta['fs']:.2f} Hz"],
        ["Duration", f"{meta['duration_s']:.3f} s"],
        ["Nyquist frequency", f"{meta['nyquist_hz']:.2f} Hz"],
        ["Frequency resolution", f"{meta['freq_resolution_hz']:.4f} Hz"],
        ["Sensor", SENSOR_NAME],
        ["Controller", CONTROLLER_NAME],
        ["Axes", "X, Y, Z"],
        ["Units", "g (gravitational acceleration)"],
    ]
    info_table = Table(info_rows, colWidths=[70 * mm, 90 * mm])
    info_table.setStyle(TableStyle([
        ("BACKGROUND", (0, 0), (-1, 0), colors.HexColor("#12233f")),
        ("TEXTCOLOR", (0, 0), (-1, 0), colors.white),
        ("FONTNAME", (0, 0), (-1, 0), "Helvetica-Bold"),
        ("FONTSIZE", (0, 0), (-1, -1), 9.5),
        ("ROWBACKGROUNDS", (0, 1), (-1, -1), [colors.white, colors.HexColor("#f2f5fa")]),
        ("GRID", (0, 0), (-1, -1), 0.4, colors.HexColor("#c9c9c9")),
        ("TOPPADDING", (0, 0), (-1, -1), 4),
        ("BOTTOMPADDING", (0, 0), (-1, -1), 4),
    ]))
    story.append(info_table)
    story.append(PageBreak())

    # -------------------- 2. SIGNAL PROCESSING --------------------
    story.append(Paragraph("2. Signal Processing Methodology", styles["SectionHeading"]))
    methodology_text = f"""
    Vibration data were acquired with an {SENSOR_NAME} accelerometer connected to an
    {CONTROLLER_NAME}, sampled at {meta['fs']:.1f} Hz. The input CSV is already calibrated/processed
    and contains acceleration channels X_g, Y_g, Z_g. The mean (DC) component of each channel was
    removed to obtain AC vibration signals X_ac, Y_ac, Z_ac. A resultant vibration signal was formed as
    R_g = sqrt(X_ac<super>2</super> + Y_ac<super>2</super> + Z_ac<super>2</super>). Frequency-domain
    analysis applies a Hann window before computing a single-sided amplitude-spectrum FFT (window-gain
    corrected) over the complete 0&ndash;Nyquist range. Power Spectral Density is estimated with
    Welch's method, and time&ndash;frequency content is examined with a short-time Fourier
    spectrogram. Significant spectral peaks are detected with an adaptive prominence threshold derived
    from each dataset's own robust noise floor (median and median-absolute-deviation of the power
    spectrum), so the detection automatically scales with the amplitude of the current dataset rather
    than relying on a fixed value. All numerical results below are directly measured quantities; the
    report performs no automatic engineering interpretation or fault classification.
    """
    story.append(Paragraph(methodology_text, styles["BodyTextJ"]))
    story.append(PageBreak())

    # -------------------- 3. TIME-DOMAIN WAVEFORMS --------------------
    story.append(Paragraph("3. Time-Domain Waveforms (Processed Signals)", styles["SectionHeading"]))
    story.append(Paragraph(
        "Full-duration waveforms of the processed acceleration channels, followed by a "
        f"{ZOOM_DURATION:.0f}-second zoom window for waveform-shape inspection.",
        styles["BodyTextJ"]))
    for col in AXIS_SIGNALS_ORIGINAL:
        _add_image_block(story, styles, time_plots[col]["full"],
                          f"{col} \u2014 full time-domain waveform.")
    for col in AXIS_SIGNALS_ORIGINAL:
        _add_image_block(story, styles, time_plots[col]["zoom"],
                          f"{col} \u2014 zoomed waveform ({ZOOM_START:.0f}\u2013{ZOOM_START+ZOOM_DURATION:.0f} s).")
    story.append(PageBreak())

    # -------------------- 4. AC VIBRATION WAVEFORMS --------------------
    story.append(Paragraph("4. AC Vibration Waveforms", styles["SectionHeading"]))
    story.append(Paragraph(
        "DC-removed AC vibration signals (X_ac, Y_ac, Z_ac) and the resultant vibration "
        "signal R_g = sqrt(X_ac\u00b2+Y_ac\u00b2+Z_ac\u00b2). Full duration and zoomed views.",
        styles["BodyTextJ"]))
    for col in AXIS_SIGNALS_AC + ["R_g"]:
        _add_image_block(story, styles, ac_plots[col]["full"],
                          f"{col} \u2014 full waveform.")
    for col in AXIS_SIGNALS_AC + ["R_g"]:
        _add_image_block(story, styles, ac_plots[col]["zoom"],
                          f"{col} \u2014 zoomed waveform ({ZOOM_START:.0f}\u2013{ZOOM_START+ZOOM_DURATION:.0f} s).")
    story.append(PageBreak())

    # -------------------- 5. TIME-DOMAIN FEATURE RESULTS --------------------
    story.append(Paragraph("5. Time-Domain Feature Results", styles["SectionHeading"]))
    story.append(Paragraph(
        "Time-domain statistical and shape features computed on X_ac, Y_ac, Z_ac and R_g. "
        "Each value is reported once and is not repeated elsewhere in this report.",
        styles["BodyTextJ"]))
    story.append(_df_to_table(time_feat_df, styles))
    story.append(PageBreak())

    # -------------------- 6. FREQUENCY-DOMAIN ANALYSIS --------------------
    story.append(Paragraph("6. Frequency-Domain Analysis (FFT)", styles["SectionHeading"]))
    story.append(Paragraph(
        f"Single-sided amplitude spectra (Hann-windowed) over the complete measured range, "
        f"0&ndash;{meta['nyquist_hz']:.1f} Hz. The {LOW_FREQ_LIMIT:.0f} Hz zoom below is a "
        "visualization aid only and is not an analysis boundary; full-spectrum peak detection "
        "was performed over the entire range (see Section 13).",
        styles["BodyTextJ"]))
    for name in ANALYSIS_SIGNALS:
        _add_image_block(story, styles, fft_plots[name]["full"],
                          f"{name} \u2014 full spectrum FFT, 0\u2013{meta['nyquist_hz']:.1f} Hz.")
    for name in ANALYSIS_SIGNALS:
        _add_image_block(story, styles, fft_plots[name]["zoom"],
                          f"{name} \u2014 FFT, 0\u2013{LOW_FREQ_LIMIT:.0f} Hz zoom (visualization only).")
    story.append(PageBreak())

    # -------------------- 7. FREQUENCY-DOMAIN FEATURE RESULTS --------------------
    story.append(Paragraph("7. Frequency-Domain Feature Results", styles["SectionHeading"]))
    story.append(Paragraph(
        "Spectral features computed from the full 0\u2013Nyquist amplitude/power spectrum "
        "(DC excluded). Reported once; not repeated elsewhere in this report.",
        styles["BodyTextJ"]))
    story.append(_df_to_table(freq_feat_df, styles, font_size=7.4))
    story.append(PageBreak())

    # -------------------- 8. POWER SPECTRAL DENSITY --------------------
    story.append(Paragraph("8. Power Spectral Density", styles["SectionHeading"]))
    story.append(Paragraph(
        f"Welch PSD (window={psd_params['window']}, nperseg={psd_params['nperseg']}, "
        f"noverlap={psd_params['noverlap']}) over the full valid range, followed by a "
        f"{LOW_FREQ_LIMIT:.0f} Hz visualization zoom. The full PSD is the primary complete-"
        "spectrum analysis.", styles["BodyTextJ"]))
    for name in ANALYSIS_SIGNALS:
        _add_image_block(story, styles, psd_plots[name]["full"],
                          f"{name} \u2014 full PSD, 0\u2013{meta['nyquist_hz']:.1f} Hz.")
    for name in ANALYSIS_SIGNALS:
        _add_image_block(story, styles, psd_plots[name]["zoom"],
                          f"{name} \u2014 PSD, 0\u2013{LOW_FREQ_LIMIT:.0f} Hz zoom.")
    story.append(PageBreak())

    # -------------------- 9. SPECTROGRAM --------------------
    story.append(Paragraph("9. Spectrogram Analysis", styles["SectionHeading"]))
    story.append(Paragraph(
        f"Short-time Fourier spectrograms (nperseg={spec_params['nperseg']}, "
        f"noverlap={spec_params['noverlap']}) over the complete available frequency range.",
        styles["BodyTextJ"]))
    for name, p in spectrogram_plots.items():
        _add_image_block(story, styles, p, f"{name} \u2014 spectrogram.")
    story.append(PageBreak())

    # -------------------- 10. AUTOCORRELATION --------------------
    story.append(Paragraph("10. Autocorrelation Analysis", styles["SectionHeading"]))
    story.append(Paragraph(
        f"Normalized autocorrelation up to a {autocorr_max_lag_s:.1f} s lag, for visual "
        "inspection of periodicity.", styles["BodyTextJ"]))
    for name, p in autocorr_plots.items():
        _add_image_block(story, styles, p, f"{name} \u2014 autocorrelation.", max_height_mm=90)
    story.append(PageBreak())

    # -------------------- 11. EMPIRICAL CDF --------------------
    story.append(Paragraph("11. Empirical CDF", styles["SectionHeading"]))
    story.append(Paragraph(
        "Empirical cumulative distribution functions computed directly from the measured "
        "data (no assumed distribution fitted).", styles["BodyTextJ"]))
    for name, p in ecdf_plots.items():
        _add_image_block(story, styles, p, f"{name} \u2014 empirical CDF.", max_height_mm=90)
    story.append(PageBreak())

    # -------------------- 12. SPECTRAL ENERGY --------------------
    story.append(Paragraph("12. Spectral Energy", styles["SectionHeading"]))
    story.append(Paragraph(
        "Spectral energy distributed across fixed visualization frequency bands (final band "
        f"extended to the actual Nyquist frequency, {meta['nyquist_hz']:.1f} Hz). Bands are "
        "for visualization only and were not used as an analysis boundary elsewhere.",
        styles["BodyTextJ"]))
    _add_image_block(story, styles, energy_plot_path,
                      "Frequency-band spectral energy, all signals.", max_height_mm=100)

    band_header = ["Band (Hz)"] + list(energy_band_tables.keys())
    ref_df = next(iter(energy_band_tables.values()))
    band_rows = [band_header]
    for i in range(len(ref_df)):
        row_label = f"{ref_df['Band_Low_Hz'].iloc[i]:.0f}\u2013{ref_df['Band_High_Hz'].iloc[i]:.1f}"
        row = [row_label]
        for name in energy_band_tables:
            row.append(_fmt(energy_band_tables[name]["Energy"].iloc[i]))
        band_rows.append(row)
    band_table = Table(band_rows, colWidths=[38 * mm] + [33 * mm] * len(energy_band_tables))
    band_table.setStyle(TableStyle([
        ("BACKGROUND", (0, 0), (-1, 0), colors.HexColor("#12233f")),
        ("TEXTCOLOR", (0, 0), (-1, 0), colors.white),
        ("FONTNAME", (0, 0), (-1, 0), "Helvetica-Bold"),
        ("FONTSIZE", (0, 0), (-1, -1), 8.4),
        ("ROWBACKGROUNDS", (0, 1), (-1, -1), [colors.white, colors.HexColor("#f2f5fa")]),
        ("GRID", (0, 0), (-1, -1), 0.4, colors.HexColor("#c9c9c9")),
        ("ALIGN", (1, 0), (-1, -1), "CENTER"),
        ("TOPPADDING", (0, 0), (-1, -1), 3.5),
        ("BOTTOMPADDING", (0, 0), (-1, -1), 3.5),
    ]))
    story.append(Spacer(1, 4 * mm))
    story.append(band_table)
    story.append(PageBreak())

    # -------------------- 13. LOCAL SPECTRAL ANALYSIS --------------------
    story.append(Paragraph("13. Local Spectral Analysis (Automatic \u00b1100 Hz Zoom)", styles["SectionHeading"]))
    story.append(Paragraph(
        "For the strongest automatically detected significant peaks in each signal's full "
        "spectrum (adaptive prominence, see Section 6/methodology), a local "
        f"\u00b1{LOCAL_BANDWIDTH_HZ:.0f} Hz window is shown with a compact annotation "
        "(frequency, amplitude, prominence). Peaks are ranked by prominence; up to "
        f"{MAX_SIGNIFICANT_PEAKS} peaks are shown per signal. \"Equivalent RPM\" values, where "
        "given, denote frequency \u00d7 60 and describe only what would follow IF that "
        "component corresponded to 1\u00d7 shaft rotation \u2014 no such correspondence is "
        "asserted by this report. Detailed peak data for all signals is saved to "
        "spectral_peaks.csv and local_spectral_analysis.csv for further analysis.",
        styles["BodyTextJ"]))

    any_peaks = False
    for name in ANALYSIS_SIGNALS:
        results = local_plot_results.get(name, [])
        if not results:
            continue
        any_peaks = True
        story.append(Paragraph(f"{name} \u2014 Significant Peaks", styles["SubHeading"]))
        for r in results:
            peak = r["peak"]
            eq_rpm = peak["frequency_hz"] * 60.0
            caption = (f"{name} rank {peak['rank']}: f = {peak['frequency_hz']:.2f} Hz, "
                       f"amplitude = {peak['amplitude_g']:.4g} g, "
                       f"prominence = {peak['prominence_amplitude']:.4g} g "
                       f"(equivalent RPM if 1\u00d7: {eq_rpm:.0f}).")
            _add_image_block(story, styles, r["path"], caption, max_height_mm=80)
    if not any_peaks:
        story.append(Paragraph(
            "No spectral peaks exceeded the adaptive significance threshold for this dataset.",
            styles["BodyTextJ"]))

    doc.build(story, onFirstPage=_on_page, onLaterPages=_on_page)
    return out_path


# ==============================================================================
# 14. MAIN
# ==============================================================================

def _print_header():
    print("=" * 60)
    print("MOTOR VIBRATION DATA ANALYSIS")
    print("=" * 60)


def main(input_file=INPUT_FILE, fs=FS):
    ensure_directories()
    _print_header()

    # ---- Load / validate / preprocess ----
    df_raw = load_data(input_file)
    df_raw = validate_data(df_raw, fs)
    df = preprocess_data(df_raw, fs)
    df = calculate_resultant(df)

    meta = dataset_metadata(df, fs, input_file)
    print(f"Input:               {meta['input_file']}")
    print(f"Samples:             {meta['n_samples']}")
    print(f"Sampling Frequency:  {meta['fs']:.2f} Hz")
    print(f"Duration:            {meta['duration_s']:.3f} s")
    print(f"Nyquist Frequency:   {meta['nyquist_hz']:.2f} Hz")
    print(f"Frequency Resolution:{meta['freq_resolution_hz']:.4f} Hz")
    print()

    signals = {name: df[name].to_numpy() for name in ANALYSIS_SIGNALS}

    # ---- Time-domain features ----
    time_feat_df = build_time_feature_table(signals)

    # ---- FFT for each analysis signal ----
    fft_data = {}
    for name, sig in signals.items():
        freqs, amp = calculate_fft(sig, fs)
        fft_data[name] = (freqs, amp)

    # ---- Adaptive peak detection (full spectrum) per signal ----
    peaks_by_signal = {}
    for name, (freqs, amp) in fft_data.items():
        peaks_by_signal[name] = detect_significant_peaks(freqs, amp, fs, meta["n_samples"])

    # ---- Frequency-domain features ----
    freq_feat_results = {}
    for name, (freqs, amp) in fft_data.items():
        freq_feat_results[name] = calculate_frequency_features(
            freqs, amp, len(peaks_by_signal[name]))
    freq_feat_df = pd.DataFrame(freq_feat_results)
    freq_feat_df.index.name = "Feature"

    # ---- Local spectral analysis per peak ----
    local_plot_results = {}
    local_rows = []
    for name, (freqs, amp) in fft_data.items():
        peaks = peaks_by_signal[name]
        local_feats = []
        for peak in peaks:
            lf = calculate_local_spectral_features(
                freqs, amp, peak, meta["nyquist_hz"], peaks)
            local_feats.append(lf)
            local_rows.append({
                "Signal": name,
                "Peak_Rank": peak["rank"],
                "Peak_Frequency_Hz": peak["frequency_hz"],
                "Peak_Amplitude_g": peak["amplitude_g"],
                "Peak_Prominence": peak["prominence_amplitude"],
                "Local_Lower_Hz": lf["Local_Lower_Hz"],
                "Local_Upper_Hz": lf["Local_Upper_Hz"],
                "Local_Energy": lf["Local_Energy"],
                "Local_Centroid_Hz": lf["Local_Centroid_Hz"],
                "Local_Bandwidth_Hz": lf["Local_Bandwidth_Hz"],
                "Local_RMS_Frequency_Hz": lf["Local_RMS_Frequency_Hz"],
            })
        results = generate_local_spectral_plots(name, freqs, amp, peaks, local_feats, PLOTS_DIR)
        local_plot_results[name] = results

    # ---- PSD ----
    psd_data = {}
    psd_params = None
    for name, sig in signals.items():
        f, pxx, params = calculate_psd(sig, fs)
        psd_data[name] = (f, pxx)
        psd_params = params

    # ---- Spectrogram ----
    spec_data = {}
    spec_params = None
    for name in AXIS_SIGNALS_AC + ["R_g"]:
        f, t, sxx, params = calculate_spectrogram(df[name].to_numpy(), fs)
        spec_data[name] = (f, t, sxx)
        spec_params = params

    # ---- Autocorrelation ----
    autocorr_data = {}
    for name, sig in signals.items():
        lags, corr = calculate_autocorrelation(sig, fs)
        autocorr_data[name] = (lags, corr)

    # ---- ECDF ----
    ecdf_data = {}
    for name, sig in signals.items():
        x_sorted, prob = calculate_ecdf(sig)
        ecdf_data[name] = (x_sorted, prob)

    # ---- Spectral energy ----
    energy_band_tables = {}
    for name, (freqs, amp) in fft_data.items():
        total_e, band_df = calculate_spectral_energy(freqs, amp, meta["nyquist_hz"])
        energy_band_tables[name] = band_df

    # ---- Terminal feature summary ----
    for name in ANALYSIS_SIGNALS:
        tf = time_feat_df[name]
        ff = freq_feat_results[name]
        print(f"{name}:")
        print(f"  RMS:               {tf['RMS']:.5g} g")
        print(f"  Absolute Peak:     {tf['Absolute_Peak']:.5g} g")
        print(f"  Peak-to-Peak:      {tf['Peak_to_Peak']:.5g} g")
        print(f"  Crest Factor:      {tf['Crest_Factor']:.5g}")
        print(f"  Kurtosis:          {tf['Kurtosis']:.5g}")
        print(f"  Dominant Frequency:{ff['Dominant_Frequency_Hz']:.3f} Hz")
        print(f"  Dominant Amplitude:{ff['Dominant_Amplitude_g']:.5g} g")
        print()

    print("SIGNIFICANT SPECTRAL PEAKS")
    print(f"{'Signal':<8}{'Rank':<6}{'Frequency (Hz)':<18}{'Amplitude (g)':<16}{'Prominence':<14}")
    for name, peaks in peaks_by_signal.items():
        for p in peaks:
            print(f"{name:<8}{p['rank']:<6}{p['frequency_hz']:<18.3f}"
                  f"{p['amplitude_g']:<16.5g}{p['prominence_amplitude']:<14.5g}")
    print()

    print("LOCAL +/-100 Hz WINDOWS")
    for name, results in local_plot_results.items():
        for r in results:
            lf = r["local_feat"]
            print(f"  {name} rank {r['peak']['rank']}: "
                  f"[{lf['Local_Lower_Hz']:.1f}, {lf['Local_Upper_Hz']:.1f}] Hz")
    print()

    # ---- Generate all plots ----
    time_plots = generate_time_plots(df, fs, ZOOM_START, ZOOM_DURATION, PLOTS_DIR)
    ac_plots = generate_ac_plots(df, fs, ZOOM_START, ZOOM_DURATION, PLOTS_DIR)
    fft_plots = generate_fft_plots(fft_data, meta["nyquist_hz"], LOW_FREQ_LIMIT, PLOTS_DIR)
    psd_plots = generate_psd_plots(psd_data, meta["nyquist_hz"], LOW_FREQ_LIMIT, PLOTS_DIR)
    spectrogram_plots = generate_spectrogram_plots(spec_data, PLOTS_DIR)
    autocorr_plots = generate_autocorrelation_plots(autocorr_data, PLOTS_DIR)
    ecdf_plots = generate_ecdf_plots(ecdf_data, PLOTS_DIR)
    energy_plot_path = generate_spectral_energy_plot(energy_band_tables, PLOTS_DIR)

    # ---- CSV outputs ----
    csv_paths = generate_csv_outputs(df, time_feat_df, freq_feat_df,
                                      peaks_by_signal, local_rows, RESULTS_DIR)

    # ---- PDF report ----
    pdf_path = generate_pdf_report(
        meta, df, time_feat_df, freq_feat_df,
        time_plots, ac_plots, fft_plots, psd_plots,
        spectrogram_plots, autocorr_plots, ecdf_plots,
        energy_plot_path, energy_band_tables,
        peaks_by_signal, local_plot_results,
        psd_params, spec_params, AUTOCORR_MAX_LAG_S,
        REPORTS_DIR,
    )

    print("=" * 60)
    print("ANALYSIS COMPLETE")
    print("=" * 60)
    print(f"PDF:")
    print(f"  {pdf_path}")
    print(f"Results:")
    for k, p in csv_paths.items():
        print(f"  {p}")
    print(f"Plots:")
    print(f"  {PLOTS_DIR}")

    return {
        "meta": meta,
        "pdf_path": pdf_path,
        "csv_paths": csv_paths,
        "plots_dir": PLOTS_DIR,
    }


if __name__ == "__main__":
    main()

# Appendix II

Figure 20 : XYZ FFT Comparison of Load Imbalance

In the Loaded Condition And Sudden Load Conditions, the load applied to the motor was: 0.239 Nm